K83LOOP - test your Marklin K83 decoders and switchpoints

���������������������������������������������������������

Copyright 1993   Rob Hamerling

K83LOOP is a simple program to test if your switchpoint decoders

(and the switchpoints themselves) are working properly. All switches

in a specified range are switched forward ('branch-off') and backward

('straight') and switched off with a certain wait-time between these

commands. This wait-time can be specified.

The COM-ports to which the Marklin Interface is supposed to be connected

can be specified as well.

Deliverables:

������������

There are 3 versions of K83LOOP, all with the same functionality:

K83LOOP.EXE   - DOS

K83LOOP2.EXE  - 16-bits version for OS/2 1.3, 2.0 or higher

K83LOOP3.EXE  - 32-bits version for OS/2 2.0 or higher

Command parameters

������������������

The following parameters can be specified with execution of K83LOOP (all

variants the same) in the following sequence:

K83LOOP  'first'  'last'  'port'  'pause'

first  -  first switch address to be tested

last   -  last address to be tested (default 256)

port   -  COM-port number (default 1)

pause  -  Wait-time in milliseconds between commands

(default 250)

Note that the 'wait-time' is actually the time a switchpoint gets its

current, until the 'switch-off' command is executed.

Only the first parameter is required. If you start K83LOOP without

parameters, you will be reminded of the parameters!

K83LOOP can be terminated while running with Ctrl-Break.

Sample report:

�������������

K83LOOP reports its activity on your display. If you call K83LOOP for

example with:

K83LOOP  253 4 2 250

you may see the following report:

switch     branch   straight  switchoff

number      time      time      time

------     -------   -------   -------

253        0.000     0.281     0.531

254        0.781     1.093     1.343

255        1.593     1.843     2.093

000        2.343     2.593     2.843    (note: 000 equivalent of 256)

001        3.093     3.343     3.593

002        3.843     4.093     4.343

003        4.593     4.843     5.093

004        5.343     5.593     5.843

Notes: - times may be rounded to the next timer-tick value!

- 'wrapping' of switchpoint addresses takes place when 'first'

address is higher than 'last'.

Internals

���������

K83 is a simple application which makes use of the MKLAPI, one of my

other products for programming the Marklin Digital system. MKLAPI is a

'driver' and high level Application Programming Interface (API).

See the MKLAPI package on the international FIDO Bulletin Board

network (in particular the Fernwood OS/2 program collection) for the

latest version.

K83LOOP is written in standard C-language. The source code of K83LOOP

will be available from me, when you register your MKLAPI package!

Feedback

��������

Comments, suggestions and problem reports are welcome. Send written

notes to:

R. Hamerling

Hogelandseweg 67

4132 CV  Vianen

The Netherlands

phone: 31.3473.72136  (voice)

Or send a message via the FIDO Bulletin Board network to:

Rob Hamerling  at  2:512/4.1098