MX5 Extended FOSSIL Functions (Int 14h)

(Also see FOSSIL.DOC for complete list of supported functions.)

AH = 0E0h (MX5 Extended FOSSIL Functions)

AL = Subfunction as Follows:

-------------------------------------------------------------

00h		Return MNP Status Block

-------------------------------------------------------------

Purpose:	Returns MNP related information.

On Entry	Register Contents

--------	-----------------

AH		E0h

AL		00h

DX		Port (0-3)

On Return	Register Contents

---------	-----------------

ES:BX		pointer to a structure as follows:

mnp_active	db	?	;0=no, 1=yes

level		db	?	;MNP level (2,4,5)

series_id	db	?	;from other MNP entity

tot_tx		dd	?	;total packets

dup_tx		dd	?	;number of duplicates

max_tx		dd	?	;max speed

tot_rx		dd	?	;total packets

dup_rx		dd	?	;number of duplicates

max_rx		dd	?	;max speed

-------------------------------------------------------------

01h		Get/Set MNP Level

-------------------------------------------------------------

Purpose:	Gets or Sets the Maximum MNP level.

On Entry	Register Contents

--------	-----------------

AH		E0h

AL		01h

BL		MNP Level

00h; None

02h; Level 2

04h; Level 4

05h; Level 5 (default)

BH		00h; Get MNP Level

01h; Set MNP Level

DX		Port (0-3)

On Return	Register Contents

---------	-----------------

BL		MNP Level (see BL above)

-------------------------------------------------------------

02h		Get/Set MNP Orig/Ans Mode

-------------------------------------------------------------

Purpose:	Gets or Sets Originate or Answer mode for

an MNP connection.

On Entry	Register Contents

--------	-----------------

AH		E0h

AL		02h

BL		Orig/Ans Mode

00h; Orig Mode (default)

01h; Answer Mode

BH		00h; Get Mode

01h; Set Mode

DX		Port (0-3)

On Return	Register Contents

---------	-----------------

BL		Mode (see BL above)

-------------------------------------------------------------

03h		Get/Set MNP Wait Tics

-------------------------------------------------------------

Purpose:	Gets or Sets number of tics (55 msecs/tic)

after Carrier Detect raises until an MNP

connection is attempted.

On Entry	Register Contents

--------	-----------------

AH		E0h

AL		03h

BL		MNP Wait Tics (default=14)

BH		0h; Get MNP Wait Tics

1h; Set MNP Wait Tics

DX		Port (0-3)

On Return	Register Contents

---------	-----------------

BL		MNP Wait Tics (see BL above)

-------------------------------------------------------------

04h		Get/Set MNP Connect Sound Level

-------------------------------------------------------------

Purpose:	Gets or Sets wether MX5 will "play"

a tune after an MNP connection.  If an

MNP connection is successful, 3 high

beeps are played.  If the MNP connection

is not successful, a high/low beep is

played.

On Entry	Register Contents

--------	-----------------

AH		E0h

AL		03h

BL		MNP Sound

00h; Sounds are not played

01h; Sounds are played (default)

BH		00h; Get Sound state

01h; Set Sound state

DX		Port (0-3)

On Return	Register Contents

---------	-----------------

BL		MNP Sound state (see BL above)

-------------------------------------------------------------

05h		Remove MX5 from Memory

-------------------------------------------------------------

Purpose:	Requests MX5 to remove itself from memory.

MX5 will de-initalize the current com port

(hanging up if online), restore all hooked

interupt vectors, and return it's MCB

address so it may be freed.

**** Note:	Place the value returned in BX into ES and

****		call DOS function 49h to release MX5's MCB

****		(see example below).

On Entry	Register Contents

--------	-----------------

AH		E0h

AL		05h

On Return	Register Contents

---------	-----------------

BX		MCB segment address or 0 if unsuccessful

Example:	mov	ah,0e0h		;tell MX5 to uninstall

mov	al,5

int	14h

cmp	bx,0

je	cant_uninstall	;jump if unsuccessful

mov	es,bx

mov	ah,49h		;free MX5's memory

int	21h

-------------------------------------------------------------

06h		Return MX5 Version Number

-------------------------------------------------------------

Purpose:	Returns the major and minor version of MX5

On Entry	Register Contents

--------	-----------------

AH		E0h

AL		06h

BX		00h

On Return	Register Contents

---------	-----------------

AH		Major version number

AL		Minor version number

BX		"MX" (4D58h)

Example:	mov	ah,0e0h		;see if MX5 is installed

mov	al,6

int	14h

cmp	bx,'MX'

jne	not_installed	;jump if MX5 not installed

-------------------------------------------------------------

07h		Wait CX Tics

-------------------------------------------------------------

Purpose:	Waits the number of tics specified in CX

On Entry	Register Contents

--------	-----------------

AH		E0h

AL		07h

CX		Tics to Wait

On Return	Register Contents

---------	-----------------

nothing