Serial Communications

Diagnostic Program

NOISE.COM

Created 11-DEC-1988

Richard B. Johnson

PROGRAM EXCHANGE (303) 440-0786

General.

This program is meant to be executed as a "live program"

from a BBS system and as an "external protocol" from a

remote-user's communications program. It will display in-

formation concerning the quality of the transmission channel

existing between the remote user and the BBS system. It may

be helpful in obtaining better quality service from the

telephone company.

Now to install this program.

If you are a BBS system operator, you create a batch file

containing the following information:

REM * Noise Program Batch file *

D:\PATH\NOISE/PORT=2/MODE=MIRROR/SPEED=19200

|     |     |      |           |     |__ optional

|     |     |      |           |________ Execute mirror mode

|     |     |      |____________________ COM port (1-4)

|     |     |___________________________ Program name

|     |_________________________________ Path to program

|_______________________________________ Drive name

You then define this as a "live program" (DOOR) in your BBS

system software configuration.

If you are a remote-user, using a communications program

like TELIX or PCPLUS, you create a batch file containing the

following information:

REM * Noise Program Batch file *

D:\PATH\NOISE/PORT=2/SPEED=19200

|     |     |      |     |

|     |     |      |     |__________ Optional

|     |     |      |________________ COM port (1-4)

|     |     |_______________________ Program name

|     |_____________________________ Path to program

|___________________________________ Drive name

You then define this batch file as an "external protocol"

DOWNLOAD utility. In TELIX, you use ALT-O to get to the cor-

rect menu. You can use "NOISE" as the protocol name. Tell

TELIX that you do NOT wish to be prompted for a file name.

How to use this program.

- 1 -

Find a BBS system that has this line-noise live program

"door" installed. Execute the live program from your com-

munications program just as though you were going to execute

any other live-program. When the BBS system advises that you

execute your local NOISE.COM program, just hit the "down-

load" key (PgDn for TELIX) and select "NOISE". The program

should begin execution and keep a running tabulation of the

amount of data sent and received, the number of bad bytes

and the percentage error-rate.

When you wish to exit the program, just hit control-break,

escape, or control-C. Control-break and escape are "cleaner"

because they don't mess up your screen with a "^C" echo and

cause the screen to be overwritten.

You can also execute this program from the DOS command line.

If you were executing a communications program, you could

"shell" to DOS and execute the same command-line that you

used within the batch file. If your program exists within

the current path, you need not type the path and drive on

the command line. If the communications adapter port in use

in COM1, you need not enter anything on the command line. In

this case, "NOISE", entered at the DOS prompt or from the

batch file, is sufficient. In most cases, you will never

have to set the baud-rate with the "SPEED" command. When the

"SPEED" command is not found, NOISE just leaves the baud-

rate alone. It remains set to whatever it was when the com-

munications program was executing, which certainly should be

the correct baud-rate.

Other usage:

NOISE has two modes of operation. The default mode is to

send a string of bytes to a remote location and record any

errors in the string being echoed back. The other mode, the

"mirror" mode, simply echoes any characters received. It

functions as though you put a connection between pins 2 and

3 of a RS-232C connector to immediately echo any bytes re-

ceived on the RX line back to the TX line. The only differ-

ence between NOISE in the "mirror" mode and a piece of wire

is that if NOISE receives a ^C, it aborts. You can see from

this that NOISE can be very useful in debugging local RS-

232C connections as well as monitoring line-noise.

NOISE.ARC contains the complete source-code and documenta-

tion. The source code shows how to operate the UARTs used in

IBM-compatible computers in interrupt-mode. Interrupt-mode

operation is necessary if you intend to use these UARTs at

speeds much in excess of 2400 baud for communications. In

particular, the code shows how to:

(1)   Set up the required interrupt vectors.

(2)   Set up the UART

- 2 -

(3)   Set up the interrupt controller.

(4)   Operate both transmit and receive in interrupt-mode.

(5)   Trap ^C and control-break so vectors may be restored.

(6)   Restore interrupt vectors before program exit.

(7)   Restore interrupt-controller mask program before exit.

(8)   Restore saved UART parameters before program exit.

Restoring the machine-state before the program exits is very

important. The major failing of many, otherwise excellent,

home-made communications programs has been that the program

may exit without restoring the machine state. The next time

a program is executed, the system will crash if the machine

state has not been correctly restored.

If you use these routines for a tutorial, remember that it

is seldom necessary to operate the UART in interrupt-mode

for transmit, only for receive. Operation of the UART in

interrupt-mode during transmit will accomplish nothing use-

ful in a communications program since the program must wait

until the host receives the characters, processes them, and

sends back a response, before the interactive user sends

more characters. Operation in interrupt-mode for receive is

important so that no incoming characters are ever lost even

if the user is typing at the keyboard, reading from a file,

etc.

How the program works.

The program sends a string of all the ANSI characters (with

^C missing) to the remote computer. The remote computer

simply echoes these characters back.

The program then compares the characters sent with those

received. If they are the same, the block-number and byte-

count is updated on the screen and the cycle continues. If a

byte is missing or garbled, the program records this fact,

calculates a running error-rate, and then continues. The

process continues until aborted by the user or until the

modem carrier fails.

If the user aborts, a string of ^C characters are sent to

the remote computer to cause the remote program to abort

also. In case of an abort for any reason, the machine-state

is restored before the program terminates.

Warranty.

This program is supplied "as-is" with no warranty whatso-

ever. The complete source-code is provided so the users may

determine for themselves if the program may serve a useful

purpose.

- finis -

- 3 -