WhatLine v 2.0              Introduction

WhatLine  is  a  utility designed to squeeze the most out of a

single  telephone  line.   Equipment  that  required dedicated

lines  can  now share a single line and save money.  Use up to

three phone numbers on one telephone line.

WhatLine with One phone number:

Your  answering machine or FAX machine can share the line with

a  modem.   The answering machine will pick up messages in the

normal  manner.   When  you  call  in  with  your  remote  PC,

WhatLine  will  have your Modem answer the line or execute any

program  you prefer.

WhatLine with Multiple phone numbers:

People,  Fax, Modem, answering machine, voice mail can share a

single  phone  line.  Dedicate  phone numbers not phone lines.

WhatLine  identifies the three different ring patterns used by

the  Identa*Ring  service  offered  by  the Telephone Company.

WhatLine  will  execute  a program based on which phone number

was  dialed.    Identa*Ring is a service mark of Bell Atlantic

and may have a different name in your area.

WhatLine's Time Event feature:

The  Time  Event  feature will execute any program at the time

specified   with   the  /T=hh:mm  start-up  option.   WhatLine

monitors  calls  as described above until a Time Event occurs,

then   your computer will execute a program(s) of your choice.

Call  remote  computer  systems and pick up or send Email when

telephone  cost  are  lowest,  do backups or run your aquarium

program  that  puts  the  little fish on the screen.  When the

Time  Event  is complete, WhatLine will wait for more incoming

calls.

Miscellaneous:

Delay  WhatLine  from passing control to a device / program so

you  can  answer the phone yourself, then use Release when you

want  WhatLine  to  handle  the  calls.   Delay only the phone

line(s)  you chose, other dialed phone numbers will be handled

by WhatLine.

The  screen  displays  activity  of  incoming calls so you can

monitor  the  time,  date  and  total number of calls for each

available phone number.

I  use  WhatLine  to  manage one phone line with two telephone

numbers.   I  run two different configurations of Watson Voice

Information  Systems by Natural Micro Systems,  one system for

my  use  and  one  for  a  client.   Each has voice mail, file

transfers  and  interactive  Voice Information Systems.  Daily

reports and disk `housekeeping' are run as a Time Event.

WhatLine v 2.0               Start up Options

Syntax    C>WHATLINE /p /QL=x /RS=x /RL=x /Dn=r /A /B /T=hh:mm

Replace lower case letters with a decimal number.

/p        The number of your Com Port  [1 to 4].  WhatLine

defaults to Com1: if omitted.

/QL=x     Used to fine tune the Quiet Long period response.

x  is a time value in seconds.  x is a minimum value

the default is 3.  This option is not normally

needed and is provided for timing variations that

may occur with some telephone systems.

/RS=x     Used to fine tune the Ring Short period response.

x is a Maximum value.  The default is 1.25

seconds.   This option is not normally needed and is

provided for timing variations that may occur with

some telephone systems.

/RL=x     Used  to fine tune the Ring Long period response.  x

is a Minimum value.  The default is 1.3 seconds.

This option is not normally needed and is provided

for timing variations that may occur with some

telephone systems.

/B        Bypass the opening screen.  Use this option if you

start WhatLine from a batch  file.

/A        Use to share one  phone number with a Answering or

Fax machine and Modem or other program.

/T=hh:mm  Time Event executes WLT.BAT at a set time.  Time

hh:mm must be in 24 hr. format.  A Time Event will

repeat daily until reset.

/Dn=r     Delays passing control for r number of rings when

phone number n is called.  May be used more than

once on the command line to delay more than one

phone number.  Has no effect if used with /A.

Files used by WhatLine

WHATLINE.EXE    The executable program file

WLT.Bat         Holds commands to run with Time Event

LINE1.BAT       Holds commands to run for phone number 1

LINE2.BAT       Holds commands to run for phone number 2

LINE3.BAT       Holds commands to run for phone number 3

WHATLINE.DOC    This file

WhatLine v 2.0              Using WhatLine

Commands  to  be executed when each phone number is dialed are

placed  in  corresponding batch files.  When phone number 1 is

dialed,   LINE1.BAT   executes.   When  all  commands  and  or

programs  in  LINE1.Bat  are  completed,  control  returns  to

WhatLine.   LINE2.BAT  and  LINE3.BAT hold the commands and or

programs   to   be   executed   for  phone  numbers  2  or  3,

respectively.   WLT.BAT  contains the commands and or programs

to  execute  as a Time Event when set with the /T=hh:mm start-

up  option.   Programs  must start-up and terminate unattended

if  you  want  control  to  return to WhatLine.   I use BitCom

from   Bit  Software  for  modem  communications,  the  script

language   is   easy   to   use   and  allows  for  unattended

operation.

When  WhatLine  is loaded the display window shows the current

time   and  the  com  port  being  monitored.   As  calls  are

processed  the  display  window  is  updated with the time and

date  of  the last call, and the total calls for each of three

possible  phone  numbers.   The display window relocates every

minute to prevent screen burn-in.

F1:Help      Show a list of options available to you, press

F1 again to clear the help line.

F2:Hold      Use this function to hold WhatLine from passing

control to the LINEn.BAT file when one or more

phone number is called.  This is good for those

times  you may want to answer the phone manually.

The display window will indicate which line(s)

is on-hold.

F3:Release   Use this function to Release a line placed on

hold with F2.  WhatLine will then pass control

according to the value set with the /Dn=r start-

up option.

F4:DOS       Use this to execute DOS commands or programs.

Type Exit to return to WhatLine.

ESC:Quit     Terminates WhatLine and returns to DOS.

To  share  one  phone  number with an answering or Fax machine

and  a  modem, you must use the /A start-up option.  Place the

commands  and/or  programs  to  be executed in LINE1.BAT.  Set

your  answering  machine  to  answer on the fourth ring,  when

the  phone  rings  the answering machine will pick up messages

in  the  normal  manner.   When  you  want  to connect to your

modem,  dial  your telephone number, let the phone ring twice,

then  hang  up.  Wait  8  seconds and call back, WhatLine will

load    your    modem   software.    When   you   finish   the

telecommunication  session,  or if you do not call back within

60  seconds,  WhatLine  will have the answering machine answer

the phone.

WhatLine v 2.0

This  program  works  with  Hayes compatible modems.  WhatLine

does  not  control the modem or change any settings.  WhatLine

monitors the 8250 UART chip for telephone line status.

If  you  are  using  a  PC with a very slow access disk drive,

using  a  RAM  drive  like  VDISK.SYS  will  speed  up overall

response  time.  If you use a RAM drive, be sure to backup any

files  that  are  changed.   If  power  is lost or your system

crashes,  all  data  on  the  RAM drive will be lost.  Use the

Time  Event  feature  to automatically backup these files to a

physical disk drive.

The  examples  in  this  document  illustrate some of the ways

that  WhatLine  can be used.  WhatLine is not limited to these

suggested  applications.  Please let me know how you are using

WhatLine,  or  what  other  features would be useful in future

releases.

This  program  is  distributed as Shareware.  You may copy and

distribute  this  program  freely.   Since  I  have been using

WhatLine,  my  monthly   telephone  savings has been more than

the $20.00 Registration fee.

Users  of  WhatLine are required to register their copy.  Upon

registration  you  will receive a disk with the latest version

of  WhatLine and some other Utilities you may find useful.  My

lawyer  asked  me to mention that I am not liable for anything

that may result from your use of this product.

Send Your $20.00 Registration fee to:

WhatLine 2.0

Advance Systems

PO Box 274

Swarthmore, Pa.  19081

2/2/90  Jim Savoia    WhatLine v 2.0 (C) 1990 Advance Systems