Quarterdeck Office Systems

Dos 5 & Quarterdeck Products

Quarterdeck Technical Note #200

by the Quarterdeck Compatibility Department

The shipping versions of all Quarterdeck products are essentially

compatible with DOS 5.  Some shipping Quarterdeck products have

already been modified with DOS 5 compatibility in mind;  others

will contain changes in future releases to enhance our DOS 5

support.  Users of Quarterdeck products who have installed DOS 5

should observe the following guidelines with our currently

shipping products:

1.  Do not use the "DOS=UMB" option or the DEVICEHIGH or LOADHIGH

statements.  Quarterdeck's LOADHI technology, available with our

QEMM and QRAM products, makes the DOS=UMB option unnecessary,

and it affects the performance of Quarterdeck products that have

not been specially modified to support it.

2.  If you are a DESQview user, do not use the DOS=HIGH option;

it results in a net loss of conventional memory inside DESQview.

If you are not a DESQview user, use QEMM386 or DOS's HIMEM.SYS

to provide the High Memory Area for DOS=HIGH to use.  Versions

of Quarterdeck's QEXT.SYS dated earlier than June 1991 do not

work with the DOS=HIGH option.

3.  Obtain DESQview 2.34 or later if you wish to run Microsoft

Windows standard mode in a DV window on a DOS 5 system.

4.  Use QEMM386, not DOS 5's EMM386.EXE, as an 80386 memory

manager if you are running DESQview on an 80386.  EMM386 cannot

provide the range of features to DESQview users that QEMM-386 can.

5.  If you load DOS 5's DOSKEY before DESQview, use its

/REINSTALL option to load it again in each DESQview window in

which you wish to use it.

6.  If you use QRAM 1.01 or earlier or QEMM-50/60 5.00 or earlier,

load QEXT.SYS before QRAM.SYS or QEMM.SYS (if you wish to use

QEXT.SYS).

7.  Don't use the DOS 5 DOSSHELL's task switcher option with

DESQview.

We believe there are difficulties running Windows 3.0

standard mode inside DESQview 2.30-2.33 on DOS 5 systems.

Various symptoms have been reported, including hangs while Windows is

loading, hangs upon exiting Windows, hangs when opening apps in

Windows, and UAEs from Windows.  DESQview 2.34 runs well with

Windows 3.0 Standard Mode on DOS 5 systems.

Manifest 1.00 crashes if loaded on a DOS 5 system.  Upgrades to

Manifest 1.01 have been available through Quarterdeck since

February 1991.

Manifest 1.01 is essentially compatible with DOS 5.  However, the

differences between DOS 5's data structures and those of earlier

versions of DOS result in a number of small inaccuracies, mostly

occurring on the DOS/Overview and DOS/Drivers screens.  Future

versions of Manifest will incorporate various changes to support

DOS 5.

It is important that users upgrading from DOS 2 or 3 to DOS 4 or 5

manually change the BUFFERS=1 line in their CONFIG.SYS file

to BUFFERS=15 (or whatever).  Otherwise, they will experience a

slowdown when BUFFERS.COM fails to load additional buffers

from the AUTOEXEC.BAT file, leaving the system with a total of

one buffer.  (BUFFERS.COM can only be used with DOS versions

2 and 3.)

The DOS 5 utility SETVER does NOT take effect when run inside

DESQview 2.33 or earlier.  DESQview 2.34 and later versions

support SETVER.

If Manifest 1.01 is made resident, then popped up over the DOS 5

DOSSHELL when the latter program is in text mode (Manifest will

not pop up over a graphics screen), the palette of DOSSHELL will

not be restored when Manifest pops down:  parts of the screen will

blink, and DOSSHELL's colours will have gone from high to

low intensity.

The DOS 5 utility DISPLAY.SYS cannot be successfully loaded into

upper memory by the LOADHI.SYS that ships with QEMM-386 5.13 and

earlier, QEMM-50/60 5.00 and earlier, and QRAM 1.01 and earlier;

it gives the error message "CON code page driver cannot be

initialized."  A new field in the DOS 5 device header that

identifies the end of memory is the reason for this failure;

future versions of LOADHI.SYS will be able to load DISPLAY.SYS

into upper memory.

80386 users who run DESQview under DOS 5's EMM386.EXE memory

scheme instead of QEMM-386 will commonly observe smaller window

sizes in DESQview, less access to expanded memory in which to run

multiple programs, a diminished ability to run programs in

background or in small windows without video conflicts, and

other problems.  It is strongly recommended in general that users

on 80386 systems use QEMM386 as their 80386 memory manager,

for the following reasons:

1.  QEMM-386 commonly provides 96K more High RAM by default

than EMM386 on non-PS/2 systems, and 32K more by default on

PS/2 systems.  EMM386 allows these areas to be included

manually, but this requires some expertise.

2.  EMM386 provides no Analysis feature.  This would help a novice

user determine whether the B000 and E000 areas could be

included.  It also lets QEMM-386 users reclaim unused

addresses in the system ROM and in other areas in high

memory, which is a great advantage to memory-hungry users.

3.  EMM386 has no OPTIMIZE program to automatically load

TSRs and device drivers into the optimum regions of upper

memory.  Novice users have difficulty getting good results

from the LOADHI feature without OPTIMIZE.

4.  EMM386 provides no control over the region of High RAM

that can be used to load programs high.

5.  DOS 5 has no equivalent to Manifest.

6.  DOS 5 has no equivalent for VIDRAM, which allows users

to extend conventional memory on EGA/VGA systems

that aren't using EGA/VGA graphics.

7.  EMM386 cannot manage ShadowRAM or Top Memory, a feature

that QEMM-386 users on limited-memory systems depend heavily

upon.  Many one-meg systems will have nothing to manage but

ShadowRAM or Top Memory.

8.  Microsoft Windows 3.0 Standard Mode will not run under EMM386.

9.  EMM386 can't map ROMs into faster RAM.

10.  EMM386 can't increase the size of conventional memory

on monochrome, Hercules, CGA, and 512K systems.

MICROSOFT's automated support line is 206/646-5103 for DOS 5.