Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

HDTEST - Hard Disk Test/Repair Utility                          ______________________________________

Copyright (1984-1989), P. R. Fletcher                           _____________________________________

All Rights Reserved                                    ___________________

Distributed by:

P. R. Fletcher (Software)

1515 West Montgomery Avenue

Rosemont

PA 19010

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

TABLE OF CONTENTS                                     _________________

User-Supported Software                                    1

Introduction                                               2

Getting Started                                            5

Hardware and Software Requirements                         7

Limitations and Restrictions                               9

Running the Programs - HDCHEK                             10

- HDTEST                             12

Using the On-Disk Pre-Boot                                16

Dealing with Bad Clusters                                 18

Running HDTEST from a .BAT file                           19

Bugs and Other Unpleasant Subjects                        20

Changes from Previous Versions                            20

Appendix A - Summary of Flag Settings                     24

Appendix B - Disk Error Messages                          26

Appendix C - Example Program Runs                         33

Registration Form                                         36

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

USER-SUPPORTED SOFTWARE                                  _______________________

Much of the software which is available for your computer has, in the

past, been produced and sold on the assumption that you are a crook

and that you will take any available opportunity to cheat software

writers and vendors out of their rightful reward. Such software was

copy-protected within an inch of its (and your computer's) life and

was priced at a level which still guaranteed its producers a profit

even if four or five illegal copies were made for each one sold.

Thankfully, prices of commercial software have (for the most part)

come down somewhat, and copy-protection is becoming much less

prevalent, but buying such software remains something of a lottery -

you often cannot be sure that the software does what you want in the

way you want it done until you have paid for it. User-supported

software (Shareware) is different. You may have got a copy of this

package free from a friend or a bulletin board or (for a nominal fee)

from a users' group or software library. In any case, you have a

perfectly legal working copy of the HDTEST package. You are encouraged

to make as many copies of the package as you like and to distribute

them to all your friends. If, after trying the program out, you find

that it is useful and want to continue to keep and use it, you must

register your copy by sending the registration fee ($35.00, unless you

are already a registered user of DIREDIT, in which case it is $25.00)

together with the completed registration form, to the address on the

form. Only your conscience can make you do this (although you are                                         ____

probably breaking the law if you don't), but bear in mind that the

distributors of "user-supported" software are making the assumption

that you are honest and willing to pay a reasonable price for a useful

program, many commercial software vendors, on the other hand, would

still apparently rather believe you are a crook. You can decide who

you would rather prove right. Registered users are entitled to

support, upgrades, and help with HDTEST-related problems (including

telephone support) for a year from the date of registration.

Page 1

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

INTRODUCTION                                       ____________

HDTEST was originally written to allow comprehensive Read/Write

testing of hard disks on IBM PCs, XTs, ATs, and "true" compatibles.

The V4.10 release removed many of the hardware restrictions imposed by

earlier (V2.xx and V3.xx) versions of the program and made it useful

for testing most MS-DOS-compatible storage devices currently in use on

IBM-compatible computers, including removable (floppy and hard) disks

and logically partitioned hard disks. The V4.34 release included BIOS-

level support for DOS 3.3 (and above) "extended devices" and more

options for the handling of totally unreadable sectors. The V4.53

release added even more extensive support for IBM-incompatible

hardware and removed the requirement that ANSI.SYS be loaded to

support a proper screen display. The current release adds support for

the large (> 32Mb) devices allowed by DOS 4.0, the ability to locate

bad spots physically (by track, head, and sector) on the disk, and

some support for the use of .BAT files to run HDTEST on multiple disks

at one "sitting".

Although many programs, including IBM's diagnostics, allow "surface

scanning" and seek testing of hard disks, none that are available to

the average user test whether the whole disk can retain the data that

is or will be written to it. HDTEST's principal "claim to fame" is the

ability thoroughly to test the magnetic media in a device which

contains data while preserving the data (although data loss is                        _________________________

unavoidable if an area which is in use has become unreadable). HDTEST                                                              ____

tests the disk much more thoroughly than programs such as DFixer,

Mace, and Norton's DT. Mace and DT, like many public domain "disk

test" programs, simply read every cluster on the disk and report on

any errors encountered.  DFixer does better, writing 4 test patterns

to every free cluster and checking that they can be read back

correctly, but it makes no attempt to test clusters that are already

in use - the very ones which contain your precious data! Disk

Technician (TM), a product which has come to market fairly recently,

is more closely comparable to HDTEST than any other disk testing

program that I know about. It is, however, much more expensive, is

(functionally, at least) copy-protected, and is not available as                                                          ___

Shareware!

In addition to its testing function, HDTEST has been reported to be

useful as an automatic disk repair utility for both hard and floppy

disks which have developed (especially) intermittent or persistent

read errors, preventing or impeding their normal use. Although HDTEST

cannot read the totally unreadable, it tries much harder that DOS does

and will almost always, in the course of its operation, allow you to

recover the entire readable portion(s) of a file with one or more                             ________

unreadable clusters in it, optionally either remapping all of what it

reads from the disk or replacing bad sectors (the default) or clusters

with NULLs. HDTEST is not an alternative to backing up your floppies                                ___

and/or hard disk - it may sometimes be able to save your neck when you                                    _________

forgot or your backup went bad too! On hard disks which have developed

intermittent read errors, running HDTEST can sometimes totally cure                                                   ___

the problem, presumably because the "flaky" sectors get rewritten and

the minor misalignments and/or weakening of the magnetic "image" which

were causing the trouble are corrected when the data is rewritten. In

its default ("slow") mode, HDTEST writes 20 different test patterns to

every cluster on the disk and checks that each pattern can be read          _____

Page 2

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

back correctly, while preserving (and eventually restoring) the

contents of the clusters that contain data.  HDTEST can even (on many

devices) detect hard disk errors which are minor enough to be

correctable (and corrected) by the disk controller's own "error-

correcting code" (ECC).  These errors are normally totally invisible

to programs running under DOS and will not usually be picked up by

other disk test utilities.  They may, however, be early signs of

problems that will become significant later, and HDTEST consequently

marks these clusters, too, as bad. As long as HDTEST can read the data

from a flaky cluster correctly once (and it tries quite hard) it will

be able to save the file of which that cluster is a part intact, while                                                                   ______

marking the cluster so that DOS does not try to use it again. The

program was designed to minimize the likelihood of user error causing

data loss on a good (or slightly bad) disk, and to be easy to use - it

could not be made very fast, since multiple bit patterns have to be

written to and read from each cluster, and this takes a significant

time even on a fast hard disk. The program does, however, have a less

sensitive "fast" test mode, which writes only two test patterns to

each block and is recommended for use when testing/repairing floppy

disks and/or to correct intermittent problems or rule out gross

problems on a hard disk.

The program can be used to test/repair single and double-sided 8- and

9-sector minifloppies (if you have plenty of patience!), as well as                                             ______

AT-format 1.2Mb minifloppies, 8" standard floppies, and 3.5"

microfloppies (with the same proviso regarding a need for patience).

HDTEST's companion program, HDCHEK, simply examines the logical

structure of the system and target device and gives a short report on

what it finds. It does not write to the disk, and is provided                                 ___

principally to allow a quick and easy, but fairly reliable, check on

the compatibility between HDTEST and a given software/hardware

configuration. As part of its report, HDCHEK notes at what level (DOS

interrupts only, INT 13, or direct entry to the BIOS) it was able to

access the target device, which has a bearing on HDTEST's ability to

detect certain types of errors. In general, if HDCHEK runs                                          __________

successfully on a system and target device, then HDTEST can be

expected to do so, although the latter program requires much more

available memory.

The HDTEST package is distributed as "User-supported Software". If you

intend to continue using the program, you are required to become a

Registered user. Send a completed copy of the Registration Form (which

is included in this documentation), together with a check or money

order for $35.00 (+ $5.00 for shipping outside the continent of North

America) to me at the address which appears on the form. Please also

add $2.50 (or equivalent) if you want the software on a 1.2 Mb HD

floppy or a 3.5" minifloppy, rather than a 360 Kb DSDD floppy, and

specify which you want on the Reg. form. Checks and money orders in

denominations other than U.S. dollars are acceptable, but please add a

further $10.00 for bank charges before using the current exchange rate

to calculate the fee in the currency in which you are paying. On

receiving your Registration, I will send you a personalised copy of

the latest, enhanced, version of the HDTEST package (currently V4.91),

and register you for a year's support. Registration entitles you to be

notified of upgrades, bug fixes, newly discovered incompatibilities,

etc., and to ask for (and get) necessary help in adapting the program

Page 3

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

to work on your hardware. I cannot guarantee to make HDTEST work with

every possible combination of hardware and software, but I will

promise to try to solve problems as they come up, and I will return

your Registration fee if I cannot support HDTEST on your system.

Support is available TO REGISTERED USERS ONLY by mail, BIXmail,                               ________________________

Compuserve EMail and telephone. Please also bear in mind that my

willingness and ability to continue to support and enhance these

programs will be directly dependent on users' willingness to pay the

Registration fee and (to some extent) inversely dependent on the

number of questions I get that could be answered by reading the docu-

mentation(!).

The HD programs were written principally in C, compiled by the

Microsoft C compiler (Version 5.10) using its "small" memory model,

and linked with code from the distributed Microsoft C object libraries

using the Microsoft LINK program (Version 4.06).  Some low-level

functions were written in assembly language and assembled by the

Microsoft MASM Macro Assembler (Version 5.10). Portions of the

distributed program are consequently Copyright (C) by Microsoft Corp.,

1985 - 1988. All rights reserved. These portions are used under the

terms of a license from Microsoft Corp.

The HD programs were written with care and have been tested on a

number of different systems.  I cannot, however, give any guarantee as

to their performance on a specific system, nor can I accept liability

for any actual or consequential loss resulting from their use. The

programs may only be used and/or copied in their original form.

Unauthorized modifications and/or the use of all or part of the

programs in other software, commercial or otherwise, are strictly

prohibited.

The HD programs owe a lot of their value to suggestions and bug

reports (more of the former than the latter, fortunately) from

Registered users. They were originally written in response to

a "challenge" issued by Roedy Green on Bix, and their further

development has also been significantly influenced by his input.

Page 4

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

GETTING STARTED                                      _______________

This section gives step by step instructions for running HDCHEK and

HDTEST on the C: drive of a standard IBM PC, XT, AT, or 100%

compatible. Subsequent sections of this documentation go into much

more detail about some of the program's other features and options,

and should help to explain and let you deal with any problems which

you may encounter. Note that you should never attempt to run HDTEST                                                  _____

under a multi-tasking or multi-user operating system, nor activate any

"pop-up" program (e.g. SideKick) while the program is running.

1) Use the DOS DISKCOPY program to make a duplicate of the

distribution disk, put the original away in a safe place, and use the

duplicate as your working disk.

2) Make sure that you have current backups of all the files on drive

C:. This is particularly important when running the program for the

first time on a given system.

3) Run DOS's CHKDSK program on drive C: and correct any errors

reported by it before proceeding further.

4) Make the drive and/or directory which contains HDCHEK.EXE and

HDTEST.EXE your current drive and/or directory.

5) Insert a formatted, non-write-protected floppy disk in drive A.

HDTEST will write an ASCII log file called HDTEST.LOG to this disk -

any other files on it will be undisturbed.

6) Give the command: "HDCHEK C:". After a certain amount of disk

activity has taken place, and certainly within a minute or two, your

screen should look like one of the examples given in Appendix C. If

any errors are reported, DO NOT PROCEED until you have read the rest                                   ______________

of the documentation and/or are sure you know what is going on.

7) If HDCHEK reported no errors, give the command "HDTEST C: A:".             ____________________________

After some disk activity, the program will prompt for replies to three

questions - press <Return> in response to each prompt. The program

will report its progress to the screen and will record any problems

encountered in the log file on the floppy disk. Depending on the size

of your DOS partition and the access time of the disk, a complete run

of HDTEST (in its normal, "slow", mode) will take between 2 and 5

hours. Your intervention will only be required if a recovered bad          _____

cluster is found in a "hidden", "system", or write-protected file,

when you will be asked to approve remapping; otherwise, bad clusters

will be automatically remapped. HDTEST can be aborted at any time by

pressing <Esc> and responding to the "Are you sure...." prompt by

pressing <Return>.

8) If HDTEST finishes testing without finding any bad clusters, it

will simply record that fact and the finishing time in the log file

and return to the DOS prompt. Otherwise, when read/write testing has

been completed, you will be prompted for permission to complete the

remapping process by updating the disk FATs and Directories.  Pressing

<Return> will allow this to take place, and you will then be returned

to the DOS prompt.

9) N.B. If HDTEST reported and remapped any bad clusters and you are             ____                                                  ___

Page 5

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

using the DOS (3.3 and above) FASTOPEN utility, or any other

directory- or file-cacheing software, you should reboot your system

before proceeding with other work on it.

10) In any event, if HDTEST reported and remapped some bad clusters,

you should read the section on "Dealing with bad clusters" later in

this documentation.

Page 6

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

HARDWARE & SOFTWARE REQUIREMENTS                             ________________________________

HDTEST and HDCHEK make a number of assumptions about the environment

in which they are operating. They will, generally speaking, abort with

an informative error message if they find one of these assumptions to

be untrue! They require MS/PC-DOS 2.x or above and (for HDTEST) up to

about 320 Kb of free memory (the exact amount required depends on the

size and other characteristics of the device being tested) to run -

the presence of more memory may (slightly) increase HDTEST's speed

when testing devices with large cluster sizes. The device to be tested

must be usable as a normal DOS device - commands such as DIR, COPY,

and CHKDSK should be valid for it and for files and subdirectories on

it. It need not, however, be bootable. Only the DOS partition(s) of a

multi-operating-system disk can be tested, but all the partitions of a

disk that is divided into multiple DOS partitions can be tested (one                                             ___

by one). This version of HDTEST fully supports DOS 3.3 (and above)

extended partitions, and will test them as thoroughly as it does

normal DOS partitions. HDTEST's support for other disk partitioning

software is provided only to the extent that DOS support is provided

by the producers of that software.

If you wish to use HDTEST to test a new hard disk which is to be                                       ____

configured to support multiple operating systems, you should first

configure it as one or more DOS partitions, test it in that

configuration, and then reconfigure it as you intend to use it. If you                                  __

have low-level disk formatting software which allows you to specify

bad sectors and/or tracks to be "locked out", you can use HDTEST's /G

option on the DOS partition(s) to produce a list of defects, which can

then be used in reformatting the disk. Note that HDTEST can only be                                                                      ____

used as a test for areas of the disk that are to be devoted to other                    ____

operating systems - when you repartition the disk, any remapping

and/or marking of bad clusters which HDTEST may have done in the DOS

partition is lost.

Ideally, for normal hard disks, the BIOS handler for interrupt 13

should be capable of handling (at least) normal disk reads and writes

(sub-functions 2 and 3) in a way that is compatible with that

documented for the XT hard disk adapter (and AT BIOS) in IBM's

technical documentation. This should be no problem for most disk

systems on IBM PCs and most compatibles.  HDTEST may, however, be

forced to use the (somewhat more limiting) DOS device interface when

used with non-bootable hard disks and/or with hard disks which require

special device drivers to be installed, and will be forced to use the

DOS device interface when dealing with disks partitioned by means

other that the DOS 3.3 (and above) utilities.

Earlier V4.xx releases of HDCHEK and HDTEST would abort if run on

small (<= 10 Mb) hard disks which had been patched to use a cluster

size smaller than the DOS standard (and 16-bit FATs). This deficiency

has been corrected in the current version, but very small (less than

1024 clusters) partitions may still cause problems if 16-bit FATs are

used (there is no good reason why such partitions should use 16-bit

FATs).

If run under DOS 3.0 or above on an IBM INT 13 compatible hard disk,

The HD programs attempt to locate and use the ROM BIOS INT 13 entry

point, rather than the normal INT 13 vector (which DOS 3.x intercepts

Page 7

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

and interferes with). By default, for machines with 8088, 8086, 80188,

80186, or equivalent CPUs the programs assume that, if a ROM is                                                              ___________

present at absolute address C8000H, it contains the hard disk BIOS and          _______                                                            ___

has initialization code starting at offset 3. This assumption is true

for all hard disks which "plug in" to the IBM PC and XT. It may not be

true for all clones which come with hard disks installed, although it

is likely to be true for most of them. Also by default, the programs

assume that the hard disk BIOS in systems with 80286 or 80386

processors is in the "system" ROM at absolute address F0000H, but

make no more specific assumptions about its location. Problems (up to               __

and including system halts) may occur if none of the special flags is

set and your system is configured in a way that is not consistent with

these assumptions. They are almost guaranteed for PC or XT systems if

the initialization code does not start at offset 3 in the ROM. If you

know that your hard disk ROM is located somewhere other than at the

default segment address and/or does not obey the "rules", you can use

the /H flag to tell the programs where to look for it. Unless you know

that your system is particularly non-IBM-compatible, however, it is

usually worth trying to run the HD programs in their default mode

first. Problem hardware should respond to one of the approaches

described in the "Using the On-Disk Preboot" section of this

documentation. The use of all the command line flags is discussed in

more detail below.

Note that, since HDTEST remaps bad clusters to free space, there is

not much point in running it on a disk which has none (as shown by DIR

or CHKDSK), since the program will abort immediately if it finds a bad

cluster.  Unless you have a really flaky disk, however, you should not

need more than a couple of hundred Kb free to remap any reasonably

probable number of bad clusters.

This version of the HD programs does not require an ANSI console

driver (such as ANSI.SYS) to be loaded. The programs will, however,

continue to function correctly if such a driver is present.

Page 8

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

LIMITATIONS AND RESTRICTIONS                               ____________________________

The current version of HDTEST still has a few (mainly theoretical)

limitations, most of which result from the way data is stored

internally by the program. None of them are "etched in stone",

although some could only be avoided at the cost of slowing down what

is already (and necessarily) far from a fast program. The known

limitations are as follows:

1) Maximum cluster size = 65,528 bytes. I do not believe that a DOS

disk can ever have clusters larger than 64 Kb, but my limit is a few

less than that and may, since sector and cluster sizes tend to run in

powers of 2, effectively limit cluster size to 32 Kb - this should be

enough for most people!.

2) DOS 4.0 and Compaq DOS 3.31 allow larger FATs than previous

versions of DOS (the FAT may be up to 128 Kbytes in extent, compared

with 64 Kbytes in previous DOS versions) Compaq DOS is not yet using                                                                         _____

FATs of this size, except on very large devices, but IBM/MS-DOS

creates FATs greater than 64Kb in extent on all partitions bigger than

about 66Mb. This version of the HDTEST package cannot handle

partitions with FATs larger than 64 Kbytes (128 sectors) in extent.

The Registered version of the package can handle devices up to the

current DOS size limit (512 Mb).

3) Because of certain limitations of the chips used to support direct

memory access (DMA, the method used to transfer data from disk to

memory and vice versa) in the IBM PC and most clones and compatibles,

HDTEST may be unable to use available memory efficiently when cluster

sizes (and the buffers needed to hold them) are large. This

inefficiency can result in "Insufficient Memory" or "Memory

Allocation" errors, even on systems with more than the minimum 320 Kb

of free memory prescribed above. The only cure for this is to free up

(or buy!) more memory (remove RAMdisks, remove resident software,

etc.).

4) The current version of HDTEST will run, in its default mode, on ATs

and most AT clones, but may not be able to detect corrected ECC errors

when run in that mode on some of the clones - a warning message will

be displayed when the program initializes itself if corrected ECC

errors are not being detected. Appropriate use of the extended form of

the /H flag or the /U flag, as described in the "Using the On-Disk

Preboot" section of this documentation, will allow detection of

corrected ECC errors on almost all hardware. The program will not                                                                        ___

detect corrected ECC errors when invoked with the /I flag (under DOS

3.0 or above) or with the /D flag.

5) For reasons which are poorly understood (at least by the author!),

HDTEST runs very slowly on Epson Equity II systems, particularly when                      ____

used to test "plug-in" hard drives of the "Hardcard" or "Flashcard"

variety. For this hardware, the estimated run times given elsewhere in

the documentation need to be multiplied by about 2.5. There is some                                       __________          ___

reason to think that this problem may be related to an excessively

"optimistic" interleave setting on (at least) some of these systems.

Page 9

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

RUNNING THE PROGRAMS                                   ____________________

Always run HDCHEK on any new hardware/software configuration before          ______

you attempt to run HDTEST (at least the first time). The latter

program has been written as conservatively as possible, but it is not

possible to predict the behavior of every conceivable combination of

hardware and operating system when dealing with a program which

bypasses most of the controls which normally protect the user from

destroying his world. HDCHEK is invoked with the command line:

HDCHEK device [/A] [/Cn | /CQ] [/D | /I | /U] [/Hnnnn[:nnnn] [/M]

"device" is the device to be tested (e.g. A:, B:, etc. - the colons

are permitted but not required). For versions of HDCHEK from V4.35,                            ___

the spaces between the device identifier and the flags (and between

different flags) are optional - the command lines "HDCHEK C:/C2/D" and

"HDCHEK C: /C2 /D" are equivalent.

The /A(T) flag is used to tell the HD program that it is being run on

an IBM AT. Setting the /A flag allows the programs to deal correctly

with a known bug in the IBM AT floppy disk controller BIOS when HDTEST

is used on 1.2 Mb "AT format" 5.25" floppy disks. If the /A flag is

set and the CPU is not an 80286 or better, you will be asked to

confirm that you wish to use AT defaults (HD BIOS at F0000H).

N.B. the definition of the /A flag changed (slightly) at V4.71 - it          ___________________________________________________________________

need now only be used if you are testing a 1.2 Mb floppy disk on a          __________________________________________________________________

"vanilla" IBM AT. It may still be used to change the default BIOS          _________________________________________________________________

segment to F0000H on a PC or XT class machine, but the use of the /H          ____________________________________________________________________

flag for this purpose is preferred.          ___________________________________

The /C flag has two variants. It is used, immediately followed by a

single decimal digit ("n") in systems which do not support IBM's

INT 11 "equipment determination" BIOS function (e.g. the Tandy 2000).

The parameter ("n") is the number of floppy drives attached to the

system, which would otherwise be determined by the program, using INT

11. The second variant should be used on Compaq computers - here, the

flag is followed by a "Q", instead of a number. This variant enables

large device support (as for IBM DOS 4.00) under Compaq DOS 3.31.

The /D(OS interrupts) flag forces HDCHEK to use the hardware-

independent DOS interrupt 25 for access to the target device. It is

usually not required, since the program is usually smart enough to          _______                                    _______

recognize devices which have to be accessed this way (e.g. logical

disk partitions other than those created as DOS 3.3 (and above)

"Extended Devices", non-bootable hard disks, some removable-media hard

disks). If the /D flag is not used, the program will issue a warning                                    ___

message if it finds it has to use the DOS interrupts.

N.B. the definition of the /D flag changed at V4.00 - the new /I flag          _____________________________________________________________________

has the same effect as the old (pre-V4.00) /D flag. Only one of these          _____________________________________________________________________

flags may be present on the command line.          _________________________________________

The /I(nterrupt 13) flag has no effect under DOS 2.x.  Under DOS 3.0

or above, it forces HDCHEK to use the existing INT 13 vector (which it

always does under DOS 2.x), and prevents the program from trying to

determine the ROM BIOS entry point; the flag should be used if you

Page 10

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

know that your system's fixed disk BIOS uses INT 13 but does not

follow IBM's "rules" or if the program crashes the system when invoked

without it.

The /H(ex BIOS address) flag is normally used (with a single numeric

argument) to tell the HD programs that the ROM BIOS of the device

being tested is located at nnnn, which is interpreted as a Hexadecimal

segment address, rather than at C800, which is the default for PC and

XT class machines, or F000, which is the default for systems using a

80286 or better CPU. There must be no space between the H and the

number. The extended form of the /H flag (e.g. /HF000:80D5) forces the

HD programs to use the given address as the ROM BIOS INT 13 entry                                               ____________________________

point. Its use is documented in more detail in the "Using the On-Disk          _____

Preboot" section of this documentation.

The /M(ulti-track read problem) flag forces HDCHEK to split up disk

accesses that would otherwise require the BIOS to read data from more

than one track in a single "bite". Its use is documented in more

detail in the "Using the On-Disk Preboot" section of this

documentation.

The /U(se INT 68) flag forces the HD programs to use the existing INT

68 vector (which must have been set up to point to an appropriate

location!) instead of locating and using the INT 13 BIOS entry point.

The use of this flag is documented in more detail in the "Using the

On-Disk Preboot" section of this documentation.

After a short delay, during most of which the specified device will be

active, a description of the device's logical structure will be disp-

layed.  After a further delay, and more disk activity, a message will

be printed giving the number of files and subdirectories found on the

disk, and the program will exit back to DOS. Note that the total

number of files and subdirectories reported by HDCHEK will be greater

than the sum of the counts reported by CHKDSK. This is because HDCHEK

counts the "." and ".." entries in subdirectories and CHKDSK doesn't.

Errors and/or problems encountered, if any, will be reported on in

some detail. A number of example screen displays and log files

produced by running HDCHEK (and HDTEST) on different systems will be

found in Appendix C at the end of this documentation. If HDCHEK tells

you that you have a bad 8088 chip, believe it - some early 8088s had a

bug which allowed them to be interrupted while changing their stack

frame*  - this is not benign, and can cause all sorts of rare,

otherwise almost inexplicable, problems. The only cure is to replace

the CPU (which is, fortunately, not a particularly expensive

proposition these days). If HDCHEK reports a discrepancy between the

two FATs, HDTEST will abort. The FIXFAT program (also in the package)

may be able to correct this problem - see FIXFAT.DOC.

_____________________________________________________________________

* See "Chips in Transition", by Bob Smith. PC Tech J. 4:4 (Apr. 1986):                                                     __________

56-69. The code which the HD programs use to identify the CPU and

diagnose this problem is adapted from the CPUID program described in

this article and includes code from subsequent, related, articles in

that magazine.

Page 11

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

Always run the DOS CHKDSK program immediately before running

HDTEST on a disk. If CHKDSK reports any problems, its /F option (or

other means) should be used to correct them before HDTEST is run.                                                      ____________________

HDTEST is invoked with the command line:

HDTEST device1 [device2[:pathname]] [optional parameters]

"optional parameters" may include one or more of the following:

[/A] [/B] [/Cn | /CQ] [/D | /I | /U] [/F] [/G[V]] [/Hnnnn[:nnnn]] [/M]

[/N] [/P] [/R[n]] [/Snnnnn] [/U] [/X] [/Yn] [/Z[n]]

"device1" is the name of the device to be tested, as for HDCHEK.

"device2" (which, if present, must be different from "device1") is the

name of a device on which a log file is to be written. If only a

device name (with or without a colon) is specified for the log file,

a file called HDTESTdn.LOG will be written to the default directory on

the specified device. "d" in this file specification is replaced by

the single-character specifier of the device being tested, while "n"

is replaced by the lowest decimal digit which results in the creation

of a new file (previous log files will not be overwritten). If all ten               ___                               ___

possible log file names for this test device have already been used,

the program will prompt for a new name. If a directory path (ending in

a backslash (\)) is specified as device2, HDTESTdn.LOG will be created

in the specified directory. If a full pathname is specified (not                                                                       ___

ending in a backslash), a file with that pathname will be opened as

the log file. In this case (only), any pre-existing file of the same

name will be overwritten. If "device2" is not specified, errors are

reported to the console terminal. The DOS console redirection facility

should not be used to redirect them elsewhere.                 ___

For versions of HDTEST from V4.35, the spaces between the device

identifier and the flags (and between different flags) are optional -

the command lines "HDTEST C:/A/F" and "HDTEST C: /A /F" are

equivalent.

The /A, /C, /D, /H, /I, /M, and /U flags have the same meaning and

effect that they do for HDCHEK. When used for HDTEST, the /D flag

forces the use of INT 25 to read from and INT 26 to write to the

target disk.

The /B(oot/Partition sector test) flag forces the program to include

the boot/partition sector (track 0, head 0, sector 1) in the DOS area

for the purposes of read/write testing. The default action is only to

perform read and seek tests on this sector, even if read/write testing

of the DOS area is specified. Since the boot/partition sector is

effectively read-only under normal circumstances (it is only written                                                                       _______

to during low-level formatting, and by FDISK and other disk          __

partitioning programs), read/write testing is less vital than for the

rest of the disk. In addition, at least one hard disk controller (a

Western Digital board which allows a single disk to be addressed as

two "physical" devices) will not allow test patterns to be written to

this sector. Certain devices (chiefly RAMdisks) appear to DOS to be

hard disks but do not, in fact, possess boot/partition sectors. On

these devices, HDTEST will report check/test results on the

Page 12

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

boot/partition sector while actually reading/writing the device's boot

sector - the latter sector will simply end up being checked/tested

twice under these circumstances.

The /F(ast mode) flag causes the program to use only two test patterns

(all 1s and all 0s) for read/write testing. It is intended for use on

floppy disks and for "quick and dirty" testing of other devices.

The /G(ive error details) flag causes the program to determine the

location of errors occurring during testing of the disk's data area

at the sector level (they are normally handled at the cluster level),

and to report on the location of all bad sectors found. The report is

written to the log device or file after all testing is finished. By                                            _____________________________

default, only an error code is given for each bad sector - if a V (for

"verbose"!) is appended to the flag, the error code is expanded to a

report similar to that given for each bad cluster. This flag is

ineffective (and ignored) if HDTEST is using DOS interrupts, either

because the /D flag was set or because the program was not able to

access the disk at the BIOS level.

The /N(o Query) flag disables the individual prompts for confirmation

which the program normally issues before remapping clusters which are

marked as "Reserved" or which are part of a file which is marked as

"System", "Hidden", or "Readonly".  The /N flag does not prevent the                                                               ___

program from prompting for confirmation before updating the disk.

The /P(rint) flag causes the log file to be directed to the PRN device

(normally LPT1), and overrides any "device2" specification.

The /R(epeat)n flag causes HDTEST to go through n complete testing

cycles. Omitting n, or setting it to zero, causes HDTEST to run

continuously until terminated by pressing <Esc>). Instead of stopping

after completing the testing of allocated data clusters, the program

immediately starts again with the DOS area. Any bad clusters found and

flagged in the first pass should be reported as "already marked as

bad" in subsequent passes. The /R flag implicitly sets the /N flag.

The /S(tart)nnnnn flag causes HDTEST to start testing occupied                                                                ________

clusters at cluster number nnnnn (expressed as a decimal number).                                                           _______

There must be no space between the "S" and the number. This option

will mainly be useful when a previous run of HDTEST was interrupted

(by pressing <Esc>), since it allows you to start where you left off.

Note that, in order to be sure that data is not remapped to a bad

area, HDTEST always tests all empty clusters before starting to test                       ______       ___

occupied clusters. For this reason, using the /S flag will not result

in a large time saving if your disk is nearly empty, and it will make

the most difference on a full, or nearly full, disk.

The /Y(es, please remap)n flag causes HDTEST to remap up to n (a

decimal number) bad clusters without prompting for confirmation. It is

effective only if HDTEST is allowed to run to completion, and is

intended to allow the program to be run unattended on multiple disks

from a .BAT file. If more than n bad clusters are found or if the

program is aborted (by pressing <Esc>), the normal prompt for

confirmation is issued before any bad clusters are remapped. See

"Running HDTEST from a .BAT file".

Page 13

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

The /X(clude bad clusters) flag causes HDTEST not to test clusters                                                        ___

that are already marked as bad by FORMAT or previous runs of HDTEST

(the default is to test all clusters every time the program is run).                                  ___

This saves some time when testing a disk with a substantial number of

known defects. The marked clusters will be reported as bad but will

not be retested. The /X flag overrides the /G flag for previously

marked clusters - no bad sector report will appear for these clusters

if both flags are used.

The /Z(ero bad sectors)n flag controls how HDTEST deals with sectors

that could not be read correctly (due to unrecoverable read errors)

when it remaps bad clusters. Versions of HDTEST up to and including

V4.10 (somewhat cavalierly) disregarded the remaining contents of the

entire cluster if one or more sectors in it were unreadable, and

remapped it to a cluster of NULLs. The default action is now to

replace only the unreadable sector(s) with NULLs, transferring the                                      _________

remaining sectors intact to the remapped cluster. There are, however,

two other options, which can be chosen by setting n to 0 or 2 (if n is

omitted, 1, or any other number, the default action is taken). Setting

n to 0 uses the old default (remaps the bad cluster to a cluster of                          ___                                      _______

NULLs), while setting n to 2 goes to the other extreme and copies all

data read (some of which is almost certainly invalid) to the remapped

cluster. It is always safe and often advisable to use the /Z2 option

provided that you do not try to use any non-ASCII files that have been          ______________________________________________________________________

remapped with unrecovered read errors and this (/Z) option. Remapped          ___________________________________________________________

ASCII files can be inspected with an editor and fixed up where

necessary - non-ASCII files (e.g. .EXE, .COM, .WKS files) may appear

to be undamaged (the program may run, the worksheet be readable, etc.)

but they can still contain small changes which will cause trouble

later, and they cannot easily be checked. The large "holes" which /Z

options 0 and 1 leave in such files usually allows it to be much more

obvious that they are defective!

After displaying its copyright message, HDTEST reads the FATs and

scans the root directory and all subdirectory trees of the specified

device before initiating a short dialogue. All questions are of the

"Yes/No" type and should be answered by typing "Y" or "N" (case does

not matter) and pressing the <Return> key.  Pressing the <Return> key

alone is equivalent to giving the default response (which appears in

square brackets at the end of each prompt).  HDTEST then proceeds to

test the specified device. By default, the DOS area (boot sector,

FATs, and root directory) is not subjected to Read/Write testing,

since failures in this area usually make themselves apparent fairly

rapidly. The user can, however, choose to run these tests on the DOS

area too, or to limit testing to read and seek tests only over the

whole partition.

In its reports to the screen, HDTEST uses the convention that read-

only testing is "Checking" while read/write testing is "Testing" - it

always "checks" the boot/partition sector and the rest of the DOS

area, but only "tests" them if requested to do so.

In "checking" or "testing" a block (sector or cluster), HDTEST first

reads it (retrying, if necessary, up to five times) and then does the

longest possible seek away from it before attempting to read it again.

"Checking" is complete at this point. "Testing" continues with the

program sequentially writing 20 (or 2, for "fast mode") different bit

Page 14

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

patterns to the block and checking that each one can be read

correctly, before rewriting the original contents of the block (and

reading it again to ensure that it has been restored correctly).

Errors that occur at any point in this process are reported and (if

the block is in the data area - errors in the DOS area are always

treated as fatal) the block is remapped. If an error is detected in a

"Reserved" cluster or in one that is part of a file which is marked as

"System", "Hidden", or "Readonly", the user is normally asked for

confirmation before the cluster is remapped, otherwise it is assumed

that remapping is desired.

HDTEST keeps a copy of the FAT and a map of the device's directory

structure in memory. A bad cluster is remapped by transferring its

contents to a free cluster and (if the bad cluster was in use)

changing all necessary FAT and directory entries in memory so that the

pointer chain which maintains the structure of the affected file or

subdirectory remains valid. The bad cluster's FAT entry is then

altered to identify it as bad to DOS. The device's FATs and                                                    ________

directories are not altered until the whole device has been scanned,

and then only after the user has confirmed that it is OK to proceed.

This means that a power failure and/or an inadvertent power-off which

occurs while the program is running will cause the loss of, at most,

one cluster of data and of no FAT or root directory information

(unless you are very unlucky and it happens after you have confirmed                          ____

that you want a damaged data area to be remapped and while this is

being done). If a power failure and/or power-off occurs while a

cluster which contains the directory information of a subdirectory is

being tested, part or all of that subdirectory will probably become

inaccessible to DOS, although the data in it will remain on the disk

and be potentially recoverable with the aid of (e.g.) the Norton

Utilities (TM).

You can safely interrupt the testing process part way through (by

pressing the <Esc> key) with no risk of leaving a partially updated

FAT or directory. If you do interrupt the program in this way, you

will be given the option of updating the disk, so that bad clusters

which were detected before <Esc> was pressed can be remapped.

Errors which occur in the program's initialization phase cause it to

abort with an error message and (sometimes) a hexadecimal error code.

The meaning of the error code is context-dependent and it is useful

only to technical support personnel. Errors detected during read-write

testing only cause the program to abort if they occur during testing

of the DOS area, or if remapping is impossible (only likely to happen

if the disk is full). In any event, a description of the error is

written to the log file (if any) and/or to the screen. This specifies

the cluster or (for the DOS area) sector number and the name of the

affected file (if any), and defines the type(s) of error which

occurred. Appendix B contains a discussion of some of the more

technical aspects of data storage on disks (floppy and hard), and

includes more information about some of the possible errors that may

be encountered.

HDTEST takes a very long time to run to completion because of the                         ____

large number of reads and writes it does for each cluster. A typical

time for full testing ("slow" mode) of an XT disk (10 Mb) would be 2

hrs; full disk testing on a 30 Mb Enhanced AT takes at least 4 hrs.

Page 15

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

Note also that using the /D or /G flags will substantially slow down

the testing of disks with significant numbers of bad spots. Using

the /X flag as well will allow the physical location of new defects to

be reported without requiring that known bad areas be exhaustively

retested.

USING THE ON-DISK PRE-BOOT                                __________________________

All IBM systems (at least non-PS/2 ones - I have little experience,

yet, with the new line of IBM hardware) - and almost all clones

capable of running IBM PC-DOS (rather than "generic" MS-DOS) should

also run the HD programs with no problems, and should allow them to

locate and use the ROM BIOS INT 13 entry point with no help from the

user. Less compatible systems have presented more problems in the

past, but this version of HDTEST is distributed with a supplemental

program (BOOTLOAD) which can be used to modify the boot sector on a

floppy disk and create a special pre-boot floppy to allow

identification and use of the "real" INT 13 entry point on almost any

MS-DOS-compatible system. Once created by BOOTLOAD, the special boot

sector will be correctly duplicated if copy the disk by using the DOS                 ____

"DISKCOPY" command - it will not be copied by a "COPY *.*" command.                                       ___

Further documentation for the BOOTLOAD program (BOOTLOAD.DOC) should

have been included in this package.

After using BOOTLOAD to create the special pre-boot disk, insert it in

the boot drive (normally drive A), then (re)boot your system as you

normally do (either by switching it on or by pressing the <Ctrl>-

<Alt>-<Del> combination or its equivalent for your hardware). After

performing its self-test, the system's BIOS will load and pass control

to the code on the boot sector of the floppy. On a normally FORMATTed

floppy disk, this code attempts to locate the MS-DOS system files at

the beginning of the disk's root directory and to load and execute

them. The HDTEST boot disk will not load DOS (even if you SYS the

disk) - it simply reads and reports on the contents of the INT 13

vector (which DOS has not yet had a chance to tamper with, since it

has not been loaded) and conditionally transfers the vector to the INT

68 location. The INT 68 location is normally unused in MS-DOS systems

- it is defined that way by IBM - and is usually loaded either with

zeroes or with a pointer to some "do-nothing" code by the BIOS

bootstrap. The HD programs use it for the "real" INT 13 vector,

normally saving the original contents of the location on entry and

restoring them on exit. This behavior is modified by specifying the /U

switch on the command line. The HDTEST boot sector code will only

transfer the "real" INT 13 BIOS entry point to the INT 68 location if

the latter contains either zeroes or the same vector as INT 69. In the                              ______        __

latter case (since there is no point in having two "real" interrupt

vectors pointing at the same place), it is assumed that both are

pointing at "do-nothing" code. If the INT 68 vector is neither 0:0 nor                                                                 _______     ___

the same as the INT 69 vector, the preboot code will leave it alone

and report that the INT 13 vector was not transferred, otherwise it

will confirm that the transfer has taken place - in any event, you

will be given an opportunity to write down the vector and can then

remove the disk and press a key to allow the boot process to complete.

If you need to boot from a floppy disk, for some reason, a new,

bootable, disk can be inserted before you press the key to continue.

Page 16

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

If the preboot reported that the INT 13 vector was successfully

transferred, you should simply specify the /U flag on the HDTEST or

HDCHEK command line, in addition to the "short" form of the /H flag                               ______________

(if the BIOS ROM is at an unusual location for the class of machine).

The requirement for the /H flag with unusual hardware allows the

program to do some additional "reasonableness checking". Unless some

other software interferes with it, the INT 68 vector will then

continue to be set up correctly for use by the HD programs until the

system is switched off or rebooted.

If the preboot was not able to transfer the vector, or if you know

that a device driver or resident program loaded by your DOS uses INT

68, you should invoke the HD programs with the "long" form of the /H

flag, specifying the full address displayed by the preboot. No error

checking is attempted in this case - if you mistype the address, the

first time the program attempts to access the disk, it will jump to a

(effectively) random location in ROM, with potentially disastrous

results. If it is feasible, use of the pre-boot disk and the /U flag

is preferable (in terms of safety, if for no other reason) to that of

the "long" form of /H.

The following example may help to clarify matters. You have a clone

system with a BIOS ROM location that is non-standard. The HDTEST boot

disk reports that the INT 13 entry point is at E000:13AC, and that the

vector has been successfully transferred. You may now run HDTEST with

the command line:

HDTEST C: A: /U /HE000                            ____

You may also, now or later, with or without prior use of the pre-boot,                                      _______________

run HDTEST with the command line:

HDTEST C: A: /HE000:13AC                         _________

Note that the /H flag is not needed with the /U flag if the segment                                   ___

address of the hard disk BIOS is the usual value for the system type                                                           ___________________

(as determined by the processor). For a system based on an 8088, 8086,

80188, or 80186 processor, this "usual" value is C800. For all other

systems, the "usual" value is F000.

One further problem may be encountered - certain hard disk BIOSes do

not exactly comply with IBM standards in dealing with disk accesses

that cross track boundaries, and will produce frequent errors when

such accesses are attempted. It was this "multi-track read" problem

which resulted in a number of previously apparently 99.99% IBM-

compatible systems failing to run IBM PC-DOS 3.3 (which was the first

version of DOS actually to attempt such accesses). IBM issued a "fix"

for this problem, in the form of a program which, when run on the

"bad" system, creates a device driver which protects the BIOS from

being asked to do something that it can't. This device driver does its

job by intercepting INT 13, and will, of course, do nothing to help

the HD programs if they are bypassing that interrupt. Use the /M flag

with the HD programs if you have a system on which PC-DOS 3.3 or above

will not run without being "fixed" as described. It forces the

programs to split up the multi-track accesses into manageable chunks -

this can be expected to cause a substantial slowing of the programs'

operation.

Page 17

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

DEALING WITH BAD CLUSTERS                                 _________________________

It is not unusual for a new hard disk to have a small, constant,                                                          _____  ________

number of bad clusters. IBM's quality control standards allow, for

example, 5% of the surface area of a disk to be bad (!). These pre-

existing bad clusters, which correspond to imperfections in the

magnetic coating on the disk, will normally be detected and marked by

the DOS FORMAT program, and HDTEST will report them as "already marked

as bad". This should not be regarded as evidence of any real problem

with the disk, although some suppliers (not including IBM) will                                                  ___

replace such disks under warranty if pressed to do so. It is also not

unusual for a disk to develop a few more defects as it ages. When                                          ___

these do develop, they are also subsequently constant in size and

position, and will be reported as "already marked as bad" by HDTEST on

runs subsequent to that during which they are first found. The

occasional appearance (and subsequent persistence) of a new bad          __________

cluster is, therefore, not necessarily evidence of serious trouble

developing.  However, if more than one or two new bad clusters are

reported by HDTEST, and especially if two consecutive runs (or passes,

if the /R flag is set) of HDTEST report different bad clusters, there                                                  _________

is a good chance that the hard disk or controller is developing a

serious problem - the disk should be backed up immediately and the

disk and/or controller sent for repair at the earliest opportunity.

New bad clusters in the data area are automatically mapped out by

HDTEST, and no further action is necessary. Bad sectors in the DOS

area can be handled in two ways:

1) If you are able to do a low-level format on your hard disk (most                                     _________

makers of "add-on" hard disks supply a method of doing this, either in

the form of a short program, or by giving instructions for running a

program in the ROM BIOS from DEBUG), back up the entire hard disk                                                           ______

(including any non-DOS partitions), run the low-level format, use

FDISK to repartition the disk, and then run the DOS FORMAT program. If

HDTEST now reports no errors in the DOS area you can restore all your

files and continue to use the disk. You should, however, use HDTEST to

test (at least) the DOS area fairly frequently for a month or so to be

sure that the problems are not going to recur.

2) If you do not have the means to do a low-level format, or if you

follow the above instructions and HDTEST continues to report errors in

the DOS area, your only other choice is to back up the DOS partition,

reconfigure the disk with FDISK so that the new DOS partition starts

above the bad area, reFORMAT it, and restore all of the old files to

the new DOS partition (which will unavoidably be a little smaller than

the old).

If the hard disk's boot/partition sector (track 0, Head 0, sector 1)

becomes bad, the disk is useless and must be replaced. This sector is

only subjected to read/write testing if the /B flag is specified on                            __________

the command line, so any reported errors (in sector -1) during normal                                                              __

testing represent a problem reading the sector and are likely to be                                      _______

significant. Problems encountered during read/write testing of this

sector do not always indicate a significant problem - certain                    ___

controller cards may not allow test patterns to be written to it,

resulting in a report of multiple errors on read/write testing.

Consequently, if multiple errors are reported on read/write testing of

Page 18

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

the boot/partition sector, but read-only testing proceeds without

problems and the rest of the disk seems to be OK, there is probably no

cause for concern.

RUNNING HDTEST FROM A .BAT FILE                              _______________________________

Versions of HDTEST from V4.75 can be run from a .BAT file (e.g. for

unattended testing of multiple disks) without the user losing the

ability to "escape" cleanly from the program. The /Y flag also allows

the user to specify a maximum number of new bad clusters for each disk

to be remapped "automatically". By using this flag, you can arrange

that the .BAT file will normally just keep going, but will pause and

wait for input if something terrible seems to have happened to a disk.

In order to run HDTEST from a .BAT file, it is necessary to prepare

both the .BAT file with your usual HDTEST command lines, generally          ____

including the /Y and /N flags, and a second ASCII file containing the                                         ___

answers to HDTEST's initial questions, one per line. DOS's input

redirection ("<") command is used to "feed" these answers to HDTEST.

If the "answer" file is called (e.g.) ANSWERS.TXT, each line in the

.BAT file will look like:

HDTEST dev1 dev2 /opt1 /opt2 <ANSWERS.TXT                                           ____________

If you do not wish to read/write test the DOS area of the disk, your

answer file can consist of three blank lines. If you do wish to                                                               __

read/write test the DOS area, the first line of the answer file can be

blank, the next two must start with a "y" and the fourth can be blank.                          ___

In order to allow HDTEST to be interrupted from the keyboard, if

necessary, the program terminates input redirection before starting

testing the disk, so anything in the answer file after the answers to                                                           _____

HDTEST's initial questions will be ignored.

On exit, HDTEST now returns a meaningful ERRORLEVEL, which can be

tested from a .BAT file by means of DOS's "IF ERRORLEVEL" syntax.

Versions of the program prior to V4.81 returned an ERRORLEVEL of 1

on aborts and 0 otherwise - HDCHEK still does this. The current

version of HDTEST normally returns an ERRORLEVEL equal to the number

of new bad clusters found. On aborts it returns 254, and, if

deliberately terminated by the user (either by answering "N" to the

last question in the introductory dialogue or by pressing and

confirming <Esc> during testing), it returns 255.

Note that this application of HDTEST is really intended for advanced

users, and I am aware that this description is somewhat cursory - more

help in using it can, if necessary, be obtained by Registered users

directly from me.

Page 19

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

BUGS AND OTHER UNPLEASANT SUBJECTS                            __________________________________

Although, like all programmers, I like to think that my product is

perfect, it is possible that there are a few bugs lurking in the

depths of the program, and likely that some combination of software

and hardware exists which HDTEST cannot figure out and/or cope with.

If you think you have found a bug and/or if you cannot get either of

the programs to work on your system, do the following:

1) Reread this documentation! Make sure that you are doing exactly

what the documentation says to do. Make sure (if you are testing a

drive with removable media) that the media is not write-protected.

2) Boot your system from a "virgin" DOS disk - preferably a copy of

your DOS distribution disk - and try to run the HD programs again. If

they run in this environment but not in your normal DOS environment,

try reconstructing the system under which they failed by adding things

one by one until the problem recurs. If you find a specific piece of

software which causes the trouble, let me know about the

incompatibility.

3) If the above approaches do not help, or if HDCHEK runs but HDTEST

bombs (the reverse should never happen), send me as detailed a

description of the problem as possible and I will do my best to

diagnose and fix it. A bug report should always include a full                                                   ______

description of the system being used (hardware and software), as well                                                         ___

as a copy of the output from a run of HDCHEK.

I can be contacted by U.S. mail at the address given earlier in the

documentation, by BIXMail to pfletcher, or by Compuserve EMail to

72255,305.

CHANGES FROM PREVIOUS VERSIONS                              ______________________________

The first version of the HD programs actually released (not including

beta-test versions) was V2.60. The programs have grown and developed

gradually, for the most part, but the pre-V4.65 releases listed below

have constituted the most significant "watersheds" - features added

and changes made between the listed releases are treated here as if                           _______

first included in the next listed release. Note that the V4.45                                ____

Shareware version of the program is not listed here - it was identical

to V4.41 except that it included the "fix" for the deficiency which

was corrected at V4.70 in the Registered version. Unless otherwise

noted, both programs were affected by the listed changes.

Version 4.84 (this version)          ____________

1) Minor change to reporting of initialization errors (HDCHEK).

Version 4.83          ____________

1) Remainder of fix for bug noted at V4.80!

Page 20

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

Version 4.81          ____________

1) Meaningful ERRORLEVEL returned on exit (see "Running HDTEST from a

.BAT file) (HDTEST).

Version 4.80          ____________

1) Bug causing the programs to fail if a subdirectory were physically

located beyond the 32 Mb boundary on a large (> 32 Mb) device

partially located and corrected.

Version 4.75          ____________

1) Log file specification capabilities enhanced (HDTEST).

2) /Y flag and .BAT file support added (HDTEST).

Version 4.71          ____________

1) Fully automatic setting of default ROM BIOS location by CPU type.

2) Meaning of and requirement for /A flag modified.

Version 4.70          ____________

1) Deficiency causing "fallback" to DOS interrupt level under some

circumstances when attempting to test "extended devices" on a second

physical hard drive fixed. The Shareware V4.45 also included this

"fix".

Version 4.66          ____________

1) Support for large devices under Compaq DOS 3.31 added (see

description of /C flag).

Version 4.65          ____________

1) DOS 4.0 support added (including BIOS-level access to large

partitions), but see notes on V4.8x.

2) Minor changes made to final screen & file output format (HDTEST).

3) Minor improvements made to memory allocation algorithms.

4) /G option added (HDTEST).

5) /X option added (HDTEST).

Page 21

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

Version 4.54          ____________

1) Deficiency in CPU recognition code (which caused fast AT clones

sometimes not to appear to be ATs) fixed.                    ___

2) ANSI.SYS requirement removed.

3) Semi-automatic CPU recognition added.

4) Pre-boot functionality, /M option, /U option, and extended

(segment:offset) form of /H option added.

Version 4.41          ____________

1) Deficiency (introduced at V4.00) causing HDCHEK to issue an

incorrect error report on an abort resulting from a FAT comparison

error corrected.

2) Minor deficiency, causing HDTEST to create an empty log file on the

specified device when both a log device and the /P option were

specified, corrected.

3) Work-around for hardware multi-sector read ECC problem (see

Appendix B) added (HDTEST).

4) Support for small (<= 10 Mb) hard disks patched to have small

clusters and 16-bit FATs added.

5) Requirement that command line option flags be separated by one or

more spaces or tabs removed.

6) /R option added (to HDTEST) for multi-pass disk testing.

Version 4.30          ____________

1) Support for BIOS-level access to DOS 3.3 extended partitions added.

2) /Z option added (to HDTEST) to control remapping of unreadable

sectors in bad clusters.

Version 4.10          ____________

1) Assorted minor changes made to screen displays.

2) /C option added to allow the programs to be used on some non-IBM

compatible MS-DOS systems.

3) /D option redefined - old /D option became new /I option.

4) /F option added (to HDTEST).

5) "Short" form of /H option added.

Page 22

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

Version 3.13          ____________

1) Assorted minor bugs and deficiencies in error reporting and

termination routines fixed (HDTEST).

2) Minor bug affecting reporting of multiple corrected ECC errors

fixed (HDTEST).

3) Disk reset added before retry on all errors (HDTEST).

4) Absolute limit on number of files and directories on device

removed.

5) Display of total sector/cluster count during testing added

(HDTEST).

6) /A option added.

7) /B option added (HDTEST).

Version 2.73          ____________

1) Bug (in V2.60) which could cause bad termination and possible

subdirectory inconsistencies if a sector which was not in use was

found to be bad, fixed (HDTEST).

2) Bug (in V2.70) which caused HDTEST to enter a terminal input loop

if ANSI.SYS was not loaded and <Esc> was entered to abort it, fixed.

3) Minor bug in V2.60 (which resulted in a failed directory scan if

the program was run on a write-protected device) fixed.

4) Internal changes made to avoid interference with expanded memory

device handlers.

5) Ability to update disk after HDTEST terminated by user (typing

<Esc>) added.

6) /S option to start checking occupied clusters at a given cluster

number added (HDTEST).

Page 23

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

APPENDIX A - SUMMARY OF FLAG SETTINGS                           _____________________________________

/A(T) - used to tell the HD program that it is being run on an IBM AT

(HDCHEK and HDTEST). /A is not equivalent to /HF000.

/B(oot/Partition sector test) - used to include the hard disk

boot/partition sector in the DOS area for the purposes of read/write

testing (HDTEST only).

/C(lone BIOS)n - used in systems which do not support the IBM BIOS's

INT 11 "equipment determination" BIOS function (e.g. the Tandy 2000).

The parameter ("n") is the number of floppy drives attached to the

system, which is otherwise determined automatically (HDCHEK and

HDTEST).

/CQ (Compaq) - used to indicate that the program is running on a

Compaq system. If DOS 3.31 (or above) is in use, large (> 32 Mb)

devices are supported (HDCHEK and HDTEST).

/D(OS interrupts) forces the program to use only DOS interrupts

(INT 25 and INT 26) for access to the target device. May not be

combined with the /I, /H, or /U flag (HDCHEK and HDTEST).

/F(ast mode) limits read/write testing to the use of two patterns

only (HDTEST only).

/G(ive error details) allows the physical disk location of bad sectors

to be identified (HDTEST only).

/H(ex BIOS address)nnnn[:nnnn] - used to tell the HD programs that the

ROM BIOS of the device being tested is located at nnnn, which is

interpreted as a Hexadecimal segment address, rather than at the

(CPU-dependent) default. The extended (segment:offset) form of the

command is used to force the programs to use the given address as the

INT 13 entry point (HDCHEK and HDTEST).

/I(nterrupt 13) - forces the program to use the existing INT 13

vector.  This is the default under DOS 2.x. May not be combined with

the /D, /H, or /U flag (HDCHEK and HDTEST).

/M(ulti-track read problem) - bypasses the multi-track read problem

which also prevents certain otherwise IBM-compatible clones from

running PC-DOS 3.3 (HDCHEK and HDTEST).

/N(o Query) - disables the individual prompts for confirmation which

the program normally issues before remapping "special" clusters - does

not prevent the program from prompting for confirmation before          ___

updating the disk (HDTEST only).

/P(rint) - causes the log file to be directed to the PRN device

(HDTEST only).

Page 24

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

/R(epeat)n - causes HDTEST to perform n complete testing cycles (or to

continue until interrupted for n=0) (HDTEST only).

/S(tart from)nnnnn - causes HDTEST to start testing occupied clusters                                                              ________

at cluster number nnnnn (expressed as a decimal number) (HDTEST only).

/U(se existing INT 68 vector) - causes the HD programs to use the

existing INT 68 vector as the INT 13 entry point (HDCHEK and HDTEST).                   ______

/X(clude marked bad sectors) - causes HDTEST to report on but not

retest clusters that are already marked as bad (HDTEST only).

/Y(es, please remap)n - allows HDTEST to update the disk by remapping

up to n new bad sectors without prompting for confirmation (HDTEST                                  _______

only).

/Z(ap bad sectors)n - controls remapping of unreadable sectors in bad

clusters (HDTEST only) - n may have one of the following values:

0 - Remap clusters with any unreadable sectors to all NULLs

1 (default) - Remap only actual unreadable sectors to NULLs

2 - Transfer all data read to the new cluster

Page 25

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

APPENDIX B - DISK ERROR MESSAGES                             ________________________________

Users of earlier versions of this software repeatedly requested that I

provide a more detailed explanation of the significance of some of the

error messages displayed by HDTEST when it encounters a bad sector or

cluster on the disk. It is not possible to do this without assuming

(or imparting) a more detailed understanding of the "nitty-gritty"

details of data storage on hard and floppy disks than most people want

or need, let alone possess. This Appendix is not, therefore, "required

reading" for the safe or effective use of the HD programs. It is

intended as a tutorial for those who would like to understand some-

thing more of what is really going on on their hard or floppy disk                                ______

drive when the little red light comes on.

For the purposes of what follows, it is (fortunately) not necessary to

know or understand exactly how changes in magnetic flux on the surface

of a hard (or floppy) disk are translated into binary data, or vice

versa. That could be the subject of another long dissertation. I will

simply take for granted that binary data, conceptually organized into

8-bit bytes, can be read and written by the disk drive hardware under

the command of its controller (which usually resides on a separate

card plugged into one of the computer's expansion slots). I will be

describing how blocks of data are physically arranged on a disk and

how the disk controller goes about ensuring that the data it reads has

not been corrupted since it was written.

I propose to start off by referring to some of the error-detection and

error-correction techniques and procedures that are applied to data

stored on magnetic media. These techniques and procedures are

necessary because, although magnetic media are very reliable and the

technology of recording and playback is well understood, there always

remains the possibility that a defect in the medium or some other

intercurrent problem may cause the sequence of bytes read back from a

disk or tape to be something other than a perfect duplicate of the

sequence that was stored. If a single copy of the raw data were all

that were recorded, there would be no way of immediately detecting

(let alone correcting) such an error. It might be brought to light

when the file that was changed caused problems when it is used by or

executed as a program, but it might not become apparent until it had

propagated itself through and caused problems or errors in multiple

other programs or files. This would clearly be an unacceptable state

of affairs. In order to prevent this from occurring, then, there

should be some means of checking data which is read in from storage

for errors, and/or (preferably) of correcting any errors which may

have occurred before any use is made of the data.                        ______

Although error checking and/or correction can (and sometimes does)

take place at any level from that of the application program down to

that of the device controller, I am only going to describe the means

that are used by hard and floppy disk controllers. When a block of

data is passed to the controller to be written to the disk, a series

of arithmetical operations is performed in sequence on all the bytes

in the block, to produce a 2-byte (for diskettes) or multi-byte (for

hard drives) code. The diskette controller implements what is known as

a cyclic redundancy check (CRC) algorithm, which is optimized to allow

detection of all single bit errors and many longer error bursts (it is          _________

reasonable to assume that errors will most probably either affect a

Page 26

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

single bit at random or, if they result from media defects, a number

of adjacent bits). The longer (at least 32-bit) error correcting code

(ECC) calculated by a hard disk controller gives enough information to

allow detection and correction of, at least, all single bit errors and                          ______________

detection of most likely longer error bursts. The CRC or ECC is saved

on disk with the data and is read back in by the controller when the

data is read. The controller then compares the saved CRC or ECC with

one newly calculated from the data. If the two are identical, the data

is assumed to be valid and a successful read is signalled. If there is

a discrepancy, a diskette controller always returns the data with a

"CRC error", since the CRC code does not contain enough information to

allow the incorrect bit or bits to be identified. On a hard disk, if

the ECC comparison suggests a correctable error, it will be corrected,

and the read call will return with a "Corrected ECC Error" code but

with (presumably) valid data in the transfer buffer. If a more

extensive error is detected, on the other hand, the call returns with

a "Uncorrectable ECC Error" code (which is the same as the CRC error

code).

The 32-bit ECC used by IBM and many compatible disk manufacturers

allows correction of single-bit errors only. Manufacturers of other,                                                 ____

usually more expensive, controllers use longer and more complex ECCs,

to allow the correction of some multi-bit errors, but the necessity to

minimize the time taken to calculate the ECC (which is part of "access

time") sets tight limits on how complex it can be. ECC error

correction is unfortunately not infallible. It is possible for a                                      ___

complex, uncorrectable error to result in the same ECC disparity as a                   __

different, correctable, error, so that invalid "corrected" data can be                                                                          ______

returned. The longer and more complex the ECC, the less likely this

becomes, but there is no way of completely avoiding this ambiguity,

short of keeping multiple copies of the data on the disk. In addition,

some hard disk controllers* apparently abort with the "corrected ECC

error" code immediately on encountering a sector containing such an                      ___________

error, even if it is not the last sector in the block specified by a                               ___

(multi-sector) read request. This non-intuitive (to say the least of

it) behavior can combine with DOS 3.x's habit of ignoring corrected                                                           ________

ECC errors to result in apparently inexplicable disk-related problems,

up to and including system crashes (if the file in which the problem

occurred contained executable code). This hardware deficiency may be

related to the problems which have plagued users of DOS 3.3 on some

(previously) apparently IBM-compatible non-IBM hardware, since these

also result from (similar but distinct) deficiencies in the clones'

handling of multi-sector disk reads. Prior to V4.36, HDTEST did retry                                                                      ___

corrected ECC errors which were reported on the first read of a

cluster, but would eventually (if the corrected error persisted)

accept the data read as valid and copy and remap it to a "good"

cluster. This could cause trouble if the controller had the "immediate

abort on corrected ECC error" problem described above and the ECC                                                                ___

error were not in the last sector of the cluster, since the sectors

after the "bad" one would not have been read, and would be lost

(actually replaced by the corresponding sectors from the previous                                                                   ________

cluster read) after remapping. On a machine with this problem, DOS

_____________________________________________________________________

* This behavior is known to occur with some early Western Digital

controllers (1002A-WX1, 1002S-WX2) but it may occur with others, too.

Page 27

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

would not, of course, have done any better than the earlier versions

of HDTEST at reading the bad cluster, and HDTEST at least warned you

that there was a problem, but the behavior described was certainly not

ideal. From V4.36, HDTEST rereads clusters in which a persistent

corrected ECC error is reported on the initial read sector by sector,                                                              ________________

to ensure that all the data in the cluster is transferred when it is                         ___

remapped. In most cases (as long as the ECC error was really                                                                ______

correctable), this will save the cluster (and the file of which it is

a part) intact.

DOS allows the programmer and/or computer user to think of and deal

with disk data in terms of files, lines, and/or records, but data is

always moved between a disk and memory a sector at a time. Any disk

read or write request passed to DOS eventually gets translated into a

command to read or write one or more sectors on the disk, commencing

at a specified head, track, and sector. A floppy disk may have either

one or two usable surfaces, corresponding to the top and bottom of the                            ________

disk. A hard disk usually has at least two surfaces, and may have

more, since multiple physical "platters" are frequently incorporated

in a single device. Each surface is normally accessed by and

corresponds to a single read/write head, which moves radially across                                             ____

it, although one surface on newer disks is often reserved for "servo"

tracks, which are written with a special pattern that allows the

controller to ensure that the read/write heads are kept exactly

aligned with the data tracks. For a single device, the (data) heads

are, by convention, numbered sequentially from zero up to one less

than the total number present. Each surface is further divided into a

number of circumferential tracks (or cylinders) and each track is                                    ______     _________

divided into a number of sectors. Again by convention, the tracks on                                   _______

each surface are also numbered sequentially from zero, with track zero

being the outermost track. The sectors on each track are normally

numbered from one to a number corresponding to the total number of                        ___

sectors on the track, but adjacent sectors do not normally have                                                        ___

consecutive sector numbers. A typical arrangement of the seventeen

sectors on a single track of an XT hard disk, for example, would be as

follows: 1,8,15,5,12,2,9,16,6,13,3,10,17,7,14,4,11. This "inter-                                                                    ______

leaving" of sectors helps to make multi-sector disk reads and writes          _______

more efficient. The controller requires a finite time to process the

data from one sector before it can start reading (or writing) the

next. During this time, of course, the disk continues to rotate and

one or more of the sectors which physically "follow" the one that was

last accessed pass under the read/write head unread. Ideally, the

interleave (number of physical sectors separating consecutively

numbered sectors) should be such that the next sector is just about to

come under the read-write head when the controller is free to deal

with it, but, because this time is not exactly predictable, it is

better to be too conservative (as IBM usually is) and make it one or

two sectors longer than necessary than to be too optimistic. Too small

an interleave value is very undesirable, because it results in the

controller having to wait almost a complete disk rotation time for

each sector it misses to come round again.

At the beginning of each disk sector is an address mark, a short,                                                     ____________

recognizable (to the controller) bit sequence containing the track and

sector number. The process of writing the address marks for each

sector on a disk is known as "low-level" or "sector" formatting. The

DOS FORMAT program does this when it is run on floppy disks, but on

Page 28

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

hard disks the DOS program merely makes sure that all required address

marks are present - they are created by the disk manufacturer or

supplier's low-level format program, which may or may not be available

to the end user. After each address mark there follows a short gap (to

allow the controller time to recognize and respond to the address

mark), then the raw data, then the CRC or ECC data. A sector normally

contains 512 bytes of raw data, but DOS (and HDTEST) can recognize and

deal appropriately with devices with non-standard sector sizes.

When a sector read/write command is passed to the disk controller, it

(the controller) causes the disk's read/write head to move (seek) to                                                                      ____

the specified track, waits a few milliseconds for it to settle down,

and then starts to "pay attention to" the data passing underneath it.

The controller locates and reads the address marks to confirm that the

read/write head is positioned at the correct track and to identify

each sector. If data is being read from the disk, the raw data and

recorded CRC or ECC from the requested sector(s) are transferred into

the disk controller's internal buffers and (assuming that no errors

are detected), thence to main memory. If data is being written, the

controller derives the error-checking data from the raw data and

writes both to the disk. Some of this process is under the direct

control of the controller hardware, but it is supervised by the

device's ROM BIOS, which is responsible for error-reporting. Errors

can occur at almost any stage of the read/write process, but (if the

disk is working at all) usually result from media defects or problems

developing in the mechanical components of the hard disk drive. Media

defects, once present, are generally constant in size and position

(although jarring a disk while it is being written to may sometimes

generate a temporary bad sector).                     _________

When HDTEST detects an error during read/write testing of a sector or

cluster which has not previously been marked as bad, a (normally)

four-line error message is printed, either to the screen (by default),

to the printer (/P option), or to the logging file (if an output

device was specified). The first line of the message simply indicates

the sector or cluster number at which the error occurred. The second

line indicates what sort of error was detected and during what phase

of read/write testing it occurred. The third line indicates whether

HDTEST was able to recover and/or remap the data from the bad sector

or cluster, and the fourth indicates either the file which a bad

cluster formed part of or that the cluster was not in use. If it was

not possible to read one or more sectors from a cluster which

contained data (unrecoverable read error), a further two-line message

is printed which notes how many sectors were unreadable and how the                                          _______

data from the bad cluster has been remapped (which depends on whether

a /Zn option was specified on the command line). If the /M flag is

not specified when HDTEST is run on a system that suffers from the

"multi-track read" problems discussed in the main part of the

documentation, multiple bad clusters may be reported with no errors                                                                    __

persisting on sector-by sector reading. These reports should disappear

if the /M flag is used.

If the "verbose" form of the /G option is used, the bad sector report

will contain the equivalent of the second and third lines of the bad

cluster report for each bad sector. Note that, if the /X option is not                         ___________________

used to prevent testing of previously flagged bad clusters, these may

give rise to spurious warnings of possible data loss in the third line

Page 29

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

of the bad sector report. If no unrecoverable bad clusters were                                                            ________

reported, such warnings can safely be ignored. If the default form of

the /G option is used, the bad sector report gives only a Hex error

code for each bad sector. The LSB of this code is the value returned

(in AH) by INT 13. The MSB of the code depends on the phase of testing

during which the error occurred. In general, if the first digit (of a

four digit code) is C or above, an unrecoverable error occurred, but

error codes FFFD through FFFF have special meanings and indicate

recovered errors.

When the program detects an error during read/write testing of a

sector in the DOS area, only the first three lines described above are

printed and the program aborts. The remainder of this Appendix

contains more details of and further information on the possible

contents of lines two and three of HDTEST's disk error message.

The second line of the disk read/write error message will usually take

the form: "[Error Type] occurred during [phase of testing]". Possible

values for [Error Type] are as follows (in alphabetical order):

Address Mark Not Found - this is a fairly common error reported for

bad blocks which are present from "scratch" on a new disk, since a

significant media defect may well not retain sector formatting data

and/or the manufacturer may have made sure that a known bad track on

the disk will not be used by deliberately not writing address marks

there. If new bad clusters are reported with this error message, it is

likely that the hard disk surface has been physically damaged (perhaps

by a shock to the drive) but possible that the controller has a

problem.

Bad Controller - this means what it says, but check that all boards

and cables are properly seated before giving up.

Bad Sector - the BIOS was instructed to read a sector number which

should not (according to its configuration data) exist on the device.

This indicates either an internal program error or that the program is

having a problem determining the physical layout of the disk. Check

the values reported by HDCHEK against those in the disk documentation.

Bad Track - analogous to "Bad Sector".

Buffer Comparison - the data read from a sector differs from that

written to it. This should not happen, since it suggests either that

the device's error-detection functionality has failed or that it has

been overwhelmed. If it does occur, the most likely causes are

controller and memory problems (in that order of likelihood).

Corrected ECC - as discussed at some length above, the data retrieved

from a hard disk was incorrect but the device's error-correction

hardware was (probably) able to correct it.

DMA Boundary Violation - should not occur (HDTEST positions its

buffers to avoid crossing absolute segment boundaries). Suggests

either an internal program error or a controller/BIOS problem.

Page 30

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

Hardware Timeout - a seek or other operation was not completed within

the maximum time allowed by the BIOS. Check connections, reseat

boards, otherwise suspect the mechanics of the disk.

Init Fail - should not occur (on a working disk).

Internal - internal program error. Make sure that your working copy of

HDTEST has not been corrupted. The disk is probably not defective.

Invalid Command - should not occur. Treat as Internal Error.

Multiple Errors - one or more additional errors occurred while HDTEST

was attempting to recover from an initial one. Multiple phase report

lines will be printed but the error types cannot be further specified.

Recovered - HDTEST recovered from the error and did not report on the

error type (should not occur).

Reset Fail - The device could not be reset (after an error). Usually

indicates major problems with the disk or controller, but check

connections, etc.

Sector Not Found - the requested sector's address mark was not found.

Likely to result from a media defect.

Seek Fail - after a seek to a track, the controller found that the

read/write head was not positioned correctly. Usually indicates a

significant problem with the (drive) hardware.

Sense Fail - the controller cannot find the drive! Check connections,

etc.

Uncorrected ECC - data error too major to be corrected. Usually a

media defect. This message will also be produced if a CRC error is

detected on a floppy disk.

Undefined - the BIOS cannot figure out what went wrong, but something

did!

Unknown - likewise!

Write Protection - HDTEST is attempting to write to a write-protected

device.

Possible forms for the rest of the second line are as follows:

... Error (unrecovered) occurred during first read - the cluster could

not be read without error in five tries before a seek to the other

"end" of the disk.

... Error occurred during first read - the error occurred during the

first read but was recovered after one or more retries.

... Error occurred during read after seek - the error occurred when

the cluster was read after a seek away.

Page 31

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

... Error occurred during pattern read/writes - the error occurred

during the pattern read/write phase of testing.

... Error occurred during final write - the error occurred when the

saved copy of the cluster was being restored to the disk.

... Error occurred during read after final write - the error occurred

when the restored cluster was being read back.

... (Buffer comparison) error occurred after final read - the copy of

the cluster read back after restoration to the disk differed from its

original memory image.

There are only three basic forms of the third line:

This error is (or "These errors are") recoverable (no data loss has

occurred). - the bad cluster was read successfully and has been

successfully restored to its original form, but it remains suspect and

will (with your permission) be remapped.

This error is unrecoverable, but remapping is possible. - the bad

cluster was read successfully but has not been restored to its                                                ___

original form. Remapping should be successful, but the data will be

lost if this is not permitted.

This error is unrecoverable, and data may be lost! - the bad cluster

was not read successfully and will be remapped as provided by the

value of the /Z switch parameter if remapping is permitted. In any

event, some data loss has almost certainly occurred.

Page 32

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher               _______________________________        _______________________________

APPENDIX C - EXAMPLE PROGRAM RUNS                   _________________________________

Screen display from successful run of HDCHEK on partition occupying the

whole of a 20 Mb hard disk:

Device C: appears to be the DOS partition of a hard disk

It contains 41531 sectors

It starts at absolute sector 17 on the disk      {THIS IS CORRECT}                                                  _______________

The total partition size is approximately 20 Mb

There are 4 heads and 17 sectors/track

Sector size is 512 bytes

Each of the 2 (16 bit) FATs occupies 41 sectors

The Root Directory occupies 32 sectors

The data area starts at DOS sector 115, and contains 10354 clusters, each

consisting of 4 sectors

The directory scan was successful; a total of 1260 files and subdirectories

was found.

Screen display from successful run of HDCHEK on a freshly FORMATted DSDD

9-sector floppy disk:

Device A: is not a hard disk - using DOS interrupts

Device A: is a floppy disk

The disk contains 720 sectors

There are 2 heads and 9 sectors/track

Sector size is 512 bytes

Each of the 2 (12 bit) FATs occupies 2 sectors

The Root Directory occupies 7 sectors

The data area starts at DOS sector 12, and contains 354 clusters, each

consisting of 2 sectors

The directory scan was successful; a total of 0 files and subdirectories

was found.

Screen display from run of HDCHEK on 20 Mb DOS partition - directory

structure bad or unrecognizable:

Device C: appears to be the DOS partition of a hard disk

It contains 41531 sectors

It starts at absolute sector 17 on the disk

The total partition size is approximately 20 Mb

There are 4 heads and 17 sectors/track

Sector size is 512 bytes

Each of the 2 (16 bit) FATs occupies 41 sectors

The Root Directory occupies 32 sectors

The data area starts at DOS sector 115, and contains 10354 clusters, each

consisting of 4 sectors

The preliminary directory scan failed.

Page 33

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher               _______________________________        _______________________________

Screen display from run of HDCHEK on 20 Mb DOS partition - FATs not

identical:

FATS are different

FAT discrepancy at byte 20F - FAT 1 has FF; FAT 2 has F8

Continue comparison (Y/N)? [Y]: n     {user input}                                _

Device C: appears to be the DOS partition of a hard disk

It contains 41531 sectors

It starts at absolute sector 17 on the disk

The total partition size is approximately 20 Mb

There are 4 heads and 17 sectors/track

Sector size is 512 bytes

Each of the 2 (16 bit) FATs occupies 41 sectors

The Root Directory occupies 32 sectors

The data area starts at DOS sector 115, and contains 10354 clusters, each

consisting of 4 sectors

Screen display from run of HDCHEK - program unable to deduce disk

structure - disk not recognizable as a DOS device:

Unrecognized media descriptor byte! - see error code

Error Code = F1      {F1 is not a media descriptor byte used by DOS}

Page 34

Documentation for HDTEST V4.84          Copyright 1988, P. R. Fletcher          _______________________________        _______________________________

Log file from successful run of HDTEST on 20 Mb DOS partition:

HDTEST Log File - Wed Jan 07 11:44:52 1986

Testing Device C:

No new bad clusters identified - HDTEST finished.

Wed Jan 07 16:10:41 1987

Log file from run of HDTEST on 20 Mb DOS partition - one bad cluster

detected and remapped:

HDTEST Log File - Mon Jan 05 21:09:32 1987

Testing Device C:

Read/Write error at cluster number 7623!!

Corrected ECC error occurred during pattern read/writes

This error is recoverable (no data loss has occurred).

Bad cluster is in file \MYFILES\DATA5\FFT1234.DAT

1 new bad clusters identified, marked, and remapped - HDTEST finished.

Tue Jan 06 05:08:04 1987

Page 35

REGISTRATION FORM - HDTEST V4.84                        ________________________________

Please complete this form and return it with a check or money order for

$35.00 (+ $5.00 for shipping outside North America) or equivalent to:

Peter R. Fletcher

1515 West Montgomery Avenue

Rosemont

PA 19010

U.S.A.

None of the information on this form will be passed on to any other

individual or commercial organization.

--------------------------------------------------------------------------

This information is required to process your Registration:

Last Name:____________________________  Other Names:______________________

Address:______________________________  Town/City:________________________

State:_____________  Post Code:____________  Telephone:_________________

(evenings/weekends)

I understand that this software is supplied without warranty, express or

implied, and agree not to hold its author liable for any direct or

consequential loss arising out of my use of it.

Signature:______________________________

-------------------------------------------------------------------------

You are requested, but not required, also to answer the following

questions:

On What type(s) of computer are you using HDTEST? _________________________

CPU (8088, 8086, etc.)? __________   Amount of RAM (kB)? ____________

Type & Size of Hard Disk(s)? ____________________________________

Make and Type of Hard Disk Controller(s)? ______________________________

Do you own a Modem?                                             Y  N

Are you a member of BIX?   Y  N        Compuserve?              Y  N

How did you obtain/hear about this program?

Did you think that the $35.00 registration fee for this program was:

Very Good Value?              About Right?             Excessively High?

Are there any other utility programs that you would like to see made

available as "User Supported Software"? What sort of programs?