TIMESYNC (TM)

USER'S GUIDE

V1.0

Copyright 1991 SolaComp Software.  All Rights Reserved.

DISCLAIMER OF WARRANTY

THIS SOFTWARE AND ANY ACCOMPANYING MANUALS ARE SOLD "AS IS"

WITHOUT WARRANTY OF ANY KIND, EXPRESSED OR IMPLIED.  THIS

INCLUDES, BUT IS NOT LIMITED TO, ANY IMPLIED WARRANTY OF

MERCHANTABILITY, SUITABILITY FOR A PARTICULAR TASK, OR ACCURACY.

THE USER ACCEPTS ALL RESPONSIBILITY FOR THE USE AND CONSEQUENCES

ARISING FROM SUCH USE.  UNDER NO CIRCUMSTANCES SHALL THE SELLER

BE LIABLE FOR ANY DAMAGES, DIRECT OR INDIRECT, RESULTING FROM USE

OF THIS SOFTWARE.

INTRODUCTION

The TimeSync program sets the computer's real time clock

with the current date and time.  This information is obtained

from a telephone call to the United States Naval Observatory

Master Clock using a modem attached to the computer.  The United

States Naval Observatory is responsible for precise time

determination.  This time is based on the frequency of emitted

radiation from a cesium laser.  The "Atomic" second is

9,192,631,770 cycles of this radiation.  The resulting time is

accurate to within a billionth of a second per day.  As an aside,

the Earth as a timepiece is only regular to thousandths of

seconds per day, and is slowing down.  Since calendars are based

on the earth's celestial position, the Master Clock must be

adjusted with "Leap Seconds" when the accumulated error is great

enough.

REQUIREMENTS

This program requires an IBM PC/XT/AT or compatible computer

with a 1200 baud modem.  A mouse is optional for the TimeSync

Set-Up program (TIMECNFG).

REGISTRATION

If you find this package useful, a registration fee of $7.50

would be greatly appreciated.  Send check or money order to:

SolaComp Software

P.O. Box 4464

Lutherville, MD 21093

INSTALLATION

The following files are included in this package:

TIMESYNC.EXE - The time setting utility

TIMECNFG.EXE - The set-up program for TIMESYNC

TIMESYNC.DOC - This file

Installation consists of setting up a directory and copying

the files into it, or simply copying all files into an existing

directory.  Running the TIMECNFG program will create a

configuration file for TimeSync.  This file contains information

such as the com port to which the modem is connected, and any

special modem control strings.

OPERATION

TIMECNFG

Before running the TimeSync program, the TimeSync

configuration program should be run.  Enter `TIMECNFG' from a DOS

prompt.  This program is menu driven, and should be self-

explanatory.  The choices in menus may be selected either using a

mouse or pressing the [ALT] key and the highlighted letter (use

the space for the triple bar symbol).  The [TAB] key is used to

move among fields on a form.  Where appropriate, the [ENTER] key

is used to accept the current values and end entry on a form.

The "Cancel" selection will terminate entry on a form without

saving any changes which may have been made.

The top level menu bar contains three selections:

�    Settings    Quit

The � (triple bar) symbol is used to select the submenu

dealing with system information.  The About selection displays

information about the program.

The Settings submenu allows system specific information to

be entered.  The selections available are UTC Offset, Com Port,

Modem Strings, Save Configuration, and Reset Configuration.

UTC Offset This leads to a submenu which allows the user to

specify the time offset between their location and UTC (Universal

Coordinated Time), also known as GMT.

The Com Port choice displays a submenu allowing the user to

specify the com port to which the modem is attached.

The Modem Strings selection will allow the user to edit the

modem command and status messages from the submenu which follows.

The documentation provided with the modem should be referenced in

changing these strings.  The default values should work for most

"AT Command" type modems.  One useful addition to the

initialization string is the command "M0" which turns off the

modem's speaker.  This may be added to the end of the

initialization string.

The XT Clock Set Command allows the user to specify the DOS

command used to set the real-time clock on a PC or XT computer.

On AT class machines the real time clock is built in, and may be

directly set.  In XT machines, the real time clock is often on an

add-on card, and each has their own method of setting the time.

The input to this selection should be the DOS command used to set

the clock (e.g. "\util\setclock").

The Save Configuration command saves the configuration in a

file in the same directory the program is stored in.

The Reset Configuration command restores the configuration

to the defaults built into the program.

The final choice is Quit.  This command terminates the set-

up program.

TIMESYNC

Once the set-up program has been run, setting the clock is

just the simple matter of typing TIMESYNC at a DOS prompt.  The

program will read the configuration information saved by TIMECNFG

and attempt to make the call.  Should any problem be encountered,

an error message will be displayed and the program will

terminate.