TIMESET (tm)

Version 6.0

A program to set computers to the world's most accurate clocks

Copyright 1990, Life Sciences Software

8925 271st N.W., Suite 112

P.O. Box 1560

Stanwood, Washington 98292

206-337-7328

(206-387-9788 after January 1, 1991)

"TIMESET" is a trademark of Life Sciences Software (tm)

CONTENTS

PERMISSION TO DISTRIBUTE  . . . . . . . . . . . . . . . . . . . . . . .   i

FEATURES OF TIMESET 6.0 . . . . . . . . . . . . . . . . . . . . . . . .  ii

SHAREWARE NOTICE  . . . . . . . . . . . . . . . . . . . . . . . . . . .  iv

Toll-Free Registration by Modem -- using TSREGSTR  . . . . . . . .  iv

INTRODUCTION  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   1

CONFIGURATION . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   2

CONFIGURING TIMESET FOR HARDWARE CLOCKS . . . . . . . . . . . . . . . .   5

"Slotless Clocks" and Tandy's SmartClock . . . . . . . . . . . . .   5

CMOS Clocks  . . . . . . . . . . . . . . . . . . . . . . . . . . .   5

Expansion Slot Add-On Clocks . . . . . . . . . . . . . . . . . . .   6

Using CLKDRV10.SYS with Board Clocks . . . . . . . . . . . . . . .   6

RUNNING THE PROGRAM . . . . . . . . . . . . . . . . . . . . . . . . . .   8

Dialing  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   8

Standard and Daylight Time . . . . . . . . . . . . . . . . . . . .   8

Monitor Color  . . . . . . . . . . . . . . . . . . . . . . . . . .   8

Pulse Generation . . . . . . . . . . . . . . . . . . . . . . . . .   9

Changing the Configuration . . . . . . . . . . . . . . . . . . . .   9

Additional Help  . . . . . . . . . . . . . . . . . . . . . . . . .  10

THE TIME DATA STRINGS . . . . . . . . . . . . . . . . . . . . . . . . .  10

The Naval Observatory's Data String  . . . . . . . . . . . . . . .  10

The NIST's Data String . . . . . . . . . . . . . . . . . . . . . .  11

LINE DELAY MEASUREMENTS . . . . . . . . . . . . . . . . . . . . . . . .  12

Line Delay Correction by NIST  . . . . . . . . . . . . . . . . . .  13

Line Delay Correction with the Naval Observatory . . . . . . . . .  14

When Should Line Delay Measurement Be Used?  . . . . . . . . . . .  15

A Note About MNP Modems  . . . . . . . . . . . . . . . . . . . . .  16

THE FINAL DISPLAY . . . . . . . . . . . . . . . . . . . . . . . . . . .  17

A FURTHER NOTE ABOUT ACCURACY . . . . . . . . . . . . . . . . . . . . .  18

TIMESET 6.0 AND THE WORDPERFECT LIBRARY . . . . . . . . . . . . . . . .  19

PROGRAMMING NOTE AND ACKNOWLEDGEMENTS . . . . . . . . . . . . . . . . .  20

CUSTOM PROGRAMMING SERVICES AVAILABLE . . . . . . . . . . . . . . . . .  21

DISCLAIMER  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  22

YOUR SUPPORT IS VERY IMPORTANT  . . . . . . . . . . . . . . . . . . . .  23

TIMESET 6 MAIL REGISTRATION FORM  . . . . . . . . . . . . . . . . . . .  24

PERMISSION TO DISTRIBUTE

All rights to TIMESET are reserved by Life Sciences Software, and all

rights to CLKDRV10.SYS are reserved by Tom Strickland, Alexandria,

Virginia.  "TIMESET" is a trademark of Life Sciences Software (tm). Both

programs are copyrighted and owned by their authors and only their authors

have the right to sell them.  However, TIMESET 6.0 and its companion files

may be distributed by anyone.  Firms involved in shareware program

distribution and charging only a reasonable fee to cover disk copying,

handling, and shipping may also distribute these files.

Please help the next person by making sure all the files are transferred

together.  The version 6.0 package must include the following (no

extraneous material added, please):

TIMESET.COM (version 6.0)

CLKDRV10.SYS (version 1.0)

TSREGSTR.COM

The READ.ME file (if one was present)

This document

- i -

FEATURES OF TIMESET 6.0

Version 6.0 of Professional TIMESET is a top-to-bottom revision and major

enhancement of the classic program for setting computer clocks with high

accuracy and precision with a telephone call to the most accurate time

sources in the world.  The program has many new features, but one of the

most notable is its ability to get time not just from one time source, but

from two.  TIMESET 6.0 does alone what used to require two different

programs from two different suppliers.  Yet the price has not changed, it's

still $35.

All the popular features of earlier TIMESET versions are still here,

including the big running time display, interactive or automatic operation,

immediate timesetting or after a countdown to zero seconds (done offline),

and rejection of noise-corrupted data strings.

The new features include:

1.   Ability to set the computer clock by calling cesium beam atomic

clocks of either the U.S. Naval Observatory in Washington, DC or

the National Institute of Standards and Technology (formerly the

National Bureau of Standards) in Boulder, Colorado.  Previous

versions could call and process time signals only from the Naval

Observatory.  Correction for line delay can be made with both

services.  If line delay is not requested, estimated delay values

are used for both services that are very close to what could be

measured for most localities in the continental United States.

2.   Ability to get line delay correction with any type of Hayes-

compatible modem.  Previous versions could get this correction

only through the Naval Observatory with modems capable of remote

digital loopback testing through the &T6 and &T0 commands.

3.   Very fast action.  It normally takes only 2-6 seconds online,

depending on the service called, if no line delay measurement is

requested.

4.   Very high accuracy.  TIMESET 6.0 even measures the time it requires to

process the received data string, and it corrects for that internal

delay before setting the time.  String processing is one of the

slowest things a computer does, and the time it takes is measurable.

Yet it's overlooked by every other timesetting program we know of.

5.   Self-configuration, with a detailed help screen available for

each configuration item and intuitive operation, making this the

least manual-dependent version of TIMESET so far.  The help

screens amount to an online tutorial, and you may not even need

the manual.  Previous versions of TIMESET needed a separate

program to configure for serial port, modem, time zone, time

season, telephone dialing procedures, type of display, etc.  All

that is self-contained in Version 6.0 and available at the press

of a function key.  The configuration is written to TIMESET's own

- ii -

source file on disk (there is no separate configuration data

file).  Most choices are made by pressing arrow keys, and editing

functions are available for the few configuration items that

require typing.  Everything is done on a single configuration

screen.

6.   The press of a single key toggles the configuration between daylight

or standard time and simultaneously adjusts the computer clock to

reflect the time season change.  Likewise, a single key lets you

override the program's video display configuration and toggle between

color and forced monochrome display (very useful for some laptop

computers).

7.   Numerous fixes, including substantial changes to eliminate

connection and synchronization problems with certain modems.  The

program now works with PC, XT, AT, PS/2, and better computers --

on serial ports 1 to 4, at dialing speeds from 1200 to 38400 bps,

with local modem speed locked or unlocked.  It automatically

detects a PS/2 and assigns the correct COM3/COM4 port addresses

for that type of computer.

8.   Greatly reduced file size -- under 50 kbytes, less than half the

combined total of 105 kbytes for TIMESET plus CONFIGTS in version

5.3 and smaller than other programs that can call only one of the

two services.

9.   Completely redesigned displays to convey more information, plus the

big running digital clock display that has always been part of the

program.  TIMESET has always been a good-looking program, because

esthetics and functionality go hand in hand.  This version is the most

esthetically pleasing yet and by far the most functional.

10.  Version 6.0 can even notify you weeks in advance when a time season

change is coming.  And each time you use it on succeeding days it can

tell you the number of days remaining before the change actually takes

effect.  On the eve of the season change it prints a special

announcement to remind you that it's almost time to reconfigure for

daylight or standard time.  On the day of the change it tells you, in

effect, "If you haven't already reconfigured, you'd better do it now"

(which you can do by merely pressing a function key, with simultaneous

automatic adjustment of your clock).

We think all these enhancements put version 6.0 of Professional TIMESET far

ahead of any other program for computer timesetting.  TIMESET 6.0 is not a

"bait" program intended to entice you to send away for something we're

holding back that's more complete, more professional, and expensive.  It IS

the complete professional version.  Nothing has been held back.  It's all

here.  You already have the maximum product.

And it's still only $35.

- iii -

SHAREWARE NOTICE

TIMESET Version 6.0 is a copyrighted commercial program developed and owned

by Peter L. Petrakis (doing business as Life Sciences Software of Stanwood,

Washington and formerly of Annapolis, Maryland) and is distributed as

shareware.

"Shareware" does not mean free and it does not mean public domain.

Shareware is a method of software distribution that lets you try a program

out and pass it along to others to try out.  Those who don't find the

program useful are free to junk it (although they could pass it along to

somebody else).  Those who like TIMESET, find it useful, and continue using

it after a reasonable trial period of 21 days are expected to pay $35 for

each copy in use (plus 8 percent sales tax for residents of Washington

State).

Payment makes you a registered user.  Registration for TIMESET version 6.0

brings benefits of technical help by telephone, a printed manual, a disk

copy of the program containing any recent changes, and a 20 percent

discount on the next major upgrade.

Organizations such as corporations, other enterprises, government agencies,

educational institutions, etc. should not expect to pay $35 for a single

copy of TIMESET then put more than one copy into use.  That is as unfair to

an author as buying a single copy of his book and using the Xerox machine

to duplicate it for the office staff.  Payment of $35 is required for each

copy of TIMESET in use within an organization.  That's not unreasonable,

and discounts are available for quantity.

If your organization requires the formality of purchase order and invoice,

don't let that be a deterrent to payment and registration; contact your

purchasing agent then let us know the procedures.  We've worked that way

with several organizations.  (Please contact us about a discount for

organizational use of more than six copies.  The discounting rate will

depend on how many additional copies your organization plans to use.)

We concede that there is no practical way to make anyone pay for shareware.

Few shareware writers have the stomach for that sort of thing anyway.

Shareware is an honor system.  And it's a valuable system that is bringing

many high-quality programs at modest prices from talented people who work

diligently to evolve and perfect their products.  It should not be

exploited through abuse or forgetfulness.

Toll-Free Registration by Modem -- using TSREGSTR

Experience suggests that many people intend to pay for a shareware program

they continue using.  But they postpone it.  Then they forget to do it.

Eventually they might not even have the registration form and the address

around anymore.  Because that's human nature, the TIMESET version 6.0

package includes a companion program, TSREGSTR, that lets you register by

modem -- with the phone call paid by us.  It's simple, fast, and

- iv -

convenient, allowing good intentions to be carried out promptly before the

memory cogs slip.  All you need to do is type in some information -- your

name, address, phone number, etc. -- and press a key to send it to our

electronic mailbox automatically.  A screen editor in TSREGSTR lets you

move around freely in the data entry field so you can make any necessary

corrections before pressing F10 to send the information.  Your phone number

gives us a chance to call you back and confirm the order and take care of

problems such as a garbled transmission.  We will then send you the latest

version of TIMESET, a printed manual, and an invoice.

It takes only a few seconds and the call is free, so there is no real

reason not to do it if you keep and use TIMESET.

To use TSREGSTR you need a 1200 or 2400 baud modem.  To keep this limited

purpose program simple, we did not provide any support for serial ports

other than COM1 and COM2 or for high-speed locked dialing (although TIMESET

itself is much more versatile and can handle COM1-COM4 and locked or

unlocked dialing speeds up to 38,400 bps).

(NOTE:  If you pass the TIMESET version 6.0 package along to others, please

be sure to include TSREGSTR.)

If you prefer to register by mail, using a check or money order, please use

the form at the end of this document.

Our voice line, if you have any general questions about registration,

licensing, quantity discounts, or the program's capabilities, is

206-337-7328 (after January 1, 1991 it will be 206-387-9788).

- v -

INTRODUCTION

One of the least reliable components in any computer is the clock.

Computer clocks are notoriously inaccurate, gaining or losing as much as

several seconds a day, and it's not unusual for them to be off by minutes

within a few weeks or even days after they are set.  This doesn't matter to

everyone who uses a computer, of course, but it matters a great deal to a

lot of people.  This includes businesses that need an accurate record of

the time of data entry, sometimes for legal reasons.  Technological and

research organizations need accurate time for numerous serious purposes

related to physical measurements.  Radio and television stations and cable

TV systems need a dependable source of time so they can switch in and out

of networks or link up with remotes at the right time.  These and many

other organizations have found previous versions of TIMESET not only useful

but necessary.

TIMESET is used in universities, NASA, the Jet Propulsion Laboratory, the

Federal Communications Commission, Federal Aviation Administration,

Veterans Administration, the military, police departments, municipal

agencies, the Federal Reserve Bank of New York, commercial banks, power

companies, Hewlett-Packard and similar high-tech companies, radio and TV

stations, cable networks, and by computer bulletin board operators,

satellite tracking hobbyists, professional and amateur astronomers, and

many people who simply want to know the time is at least close to what

their computer says it is.

There are many TIMESET users of all kinds in the United States and abroad

who are dismayed that the clock in their expensive computer, which is

driven by a high-frequency crystal-controlled oscillator, cannot keep time

nearly as well as a ten dollar wind-up watch from K-Mart.  They think a

computer should provide a standard for setting other timepieces, not the

other way around, and they are right.  Nor are all of them necessarily

interested in split-second accuracy for their computer clocks; many users

simply want a convenient way to set their computer to a standard they know

is right.  TIMESET provides that convenience, and the Naval Observatory and

NIST clocks are as right as any clocks can be.

The first version of TIMESET was released to computer bulletin boards in

July 1987.  It was to be the only version (it didn't even have a version

number), but the response was astonishing.  Phone calls came day and (alas)

night from people all over the country who were either delighted with the

program or had problems getting it to work with their systems, particularly

their modems.  That interest has stimulated continued development ever

since, and successive versions have been released to make the program work

reliably for more and more people and to add new features, many of them

suggested by users.  It was very gratifying to see Version 5.3, the most

recent version until now, written up by PC/Computing Magazine in its

October 1990 issue in an article devoted to good ways to enhance a DOS

computer's operating environment.

Version 6.0 is a top-to-bottom revision that includes, for the first time,

the ability to get atomic time from either of the world's two preeminent

- 1 -

sources for timing by telephone -- the National Institute of Standards and

Technology (NIST) and the U.S. Naval Observatory.  There are programs,

including previous versions of TIMESET, that can call one or the other of

these two services, but TIMESET 6.0 so far appears to be the only one that

can dial both of them (we expect to be corrected if that's not right or

ceases to be so) and make full use of the capabilities of each.  Thus users

from one end of the country to the other can now reduce phone costs by

using the nearest service.

Users of previous versions will notice numerous other changes.  The program

looks and feels completely different.  Though it has more features, it is

much smaller on disk.  Configuration procedures are now built into the

program itself (there is no external configuration data file and no

external configuration program) and are very easy to do.  You can enter

configuration mode at any time to change things by pressing a function key

at the main menu.  Help for each configuration item is available by

pressing another function key when the item is highlighted on the

configuration screen.

You can now toggle between standard and daylight time configuration by

pressing a function key at the main menu -- and it simultaneously sets the

computer clock one hour earlier or later depending on the season.  You can

generate a usable pulse for synchronizing an external device at the instant

of timesetting by pressing another function key before dialing.  Command

line options are displayed by typing TIMESET followed by a space and a

question mark at the DOS command.  A quick overview of internal command

options is available by pressing a function key at the main menu.  Finally,

TIMESET 6.0 can measure and correct for line delays with either the Naval

Observatory or the National Institute of Standards and Technology.

CONFIGURATION

Because configuration data for TIMESET 6.0 are written directly to its

source file on disk, the program needs to know where its source file is; it

reads its configuration each time it's loaded and each time you change the

configuration.  With DOS versions 3 and up it's no problem; those versions

allow TIMESET to find out where it came from, making it possible to

configure or run the program from any drive and directory on the system

path (which is set up with the DOS PATH command).  That's not the case with

earlier versions of DOS.  With those DOS versions, TIMESET 6.0 needs to be

told where its source file is.  That's done by using the DOS SET command to

put its location in the DOS environment, like this:

SET TIMESET=[drive]:\[directory\]TIMESET.COM

The drive, directory (if any), and ".COM" are essential.  If you're using a

version of DOS earlier than 3.0 and are going to be using TIMESET 6.0

regularly, it makes sense to put that command in your AUTOEXEC.BAT file,

which should also include a path to its location using the PATH command.

Otherwise you'll always have to be in the drive and directory where TIMESET

is located before you can use it.

- 2 -

THIS IS IMPORTANT!!:  If the copy of TIMESET version 6.0 that you received

has never been configured before, it will automatically load in

configuration mode so you can tailor it to your own system.  If you got it

from a computer bulletin board, however, the chances are it will have

already been configured, probably by the bulletin board operator who tested

it before making it available for downloading or by the person who uploaded

it to the bulletin board.  In that case, it will load in operating mode.

In fact, if it was configured to run in non-prompted (automatic) mode it

will immediately start dialing.  For this reason, if you obtained TIMESET

6.0 from a computer bulletin board or other indirect source, you should put

/C on the command line the first time you run it, like this:

TIMESET /C

This will force the program to go to configuration mode as soon as it's

loaded so you can tailor the configuration to your own system.  Using /C

will not be necessary if your copy of TIMESET 6.0 came directly from Life

Sciences Software, because we do not preconfigure the program before

shipping it.

The configuration screen comes up with values for phone numbers, time

season, serial port, baud rate, etc. already listed.  Please understand

that if TIMESET 6.0 has never been configured before, those values are not

yet in the program file's configuration field; they are what WILL be

written to that field if you press F10.  On the other hand, if TIMESET has

already been configured, they represent the actual current configuration

that the program read from its own disk source.

After the program has been configured, you can always enter configuration

mode to check or change your settings by pressing F2 at the main menu.  If

you are just checking and not changing anything, you don't have to press

F10 to resave what's on the screen.  Instead, you can get back to the main

program by pressing F2 again.  If you make a change, you should of course

press F10 to save it.

If you are configuring for the first time and the values you see are what

you want, press F10 to save them to TIMESET.COM on your disk.  There is a

good chance, however, that you'll need to change some of the default values

so they'll correspond to your actual computer/modem setup.  That's what

configuration of version 6.0 is mostly about -- changing configuration

values that are already displayed.  It's very easy to do, and it's likely

that you'll only need to make minor changes in two or three items.

Each configuration item has its own help screen, which can be displayed by

pressing F1 when that item is highlighted.  The highlighting can be moved

up or down by pressing the up or down arrow key on your keypad.  Before

doing any configuring, we strongly recommend that you use the up and down

arrow keys to go through the entire list of configuration items one by one

and press F1 to get the help screen for each.  That will give you a good

overview of what the configuration procedure is about.  The help screens

collectively amount to an online configuration manual.  In a way, they ARE

- 3 -

the manual, because few particulars are discussed here.  Version 6.0 has

been designed to reduce the need to consult a printed manual to the

minimum.  As result, this manual can devote more attention to some

theoretical matters that are worth knowing about.

Everything is done on a single screen, mostly by pressing the right, left,

up, and down arrow keys on your keyboard.  Pressing the up or down arrow

key highlights a new configuration item.  The right and left arrow keys

toggle the options that are available for a given configuration item.  For

example, on the configuration line for baud rate, you can go upward from

1200 baud -- 1200, 2400, 4800, etc. -- to 38400 baud by repeatedly pressing

the right arrow key.  Pressing the left arrow key takes you in the opposite

direction.

Only four of the configuration items involve typing anything, and even that

might not be necessary if the default or preexisting settings for those

items are what you want.  Three of the items that may require typing or

editing pertain to telephone numbers, the fourth pertains to the user's

modem initialization string.  All the other items are toggled with the

right-left arrow keys.  To change the phone numbers or the modem

initialization string, you have to get into editing mode.  That's done by

pressing Return when the editable configuration line is highlighted (a

message also appears telling when a line can be edited).  In editing mode

you can insert characters or delete or type over existing characters.  A

description of editing functions (inserting, overstriking, deleting, cursor

movement, etc.) pops onto the screen at the right as soon as you press

Return, so not much needs to be said about them here.

For example, the default phone number for the Naval Observatory is shown as

12026530351.  If you live in Washington, D.C., the Naval Observatory is a

local call and you obviously need to get rid of 1202.  You do that by

scrolling to the appropriate line with the up or down arrow key, pressing

Return, pressing the Delete key four times, then pressing Return again.  Or

suppose your company requires you to add an account number to the end of a

long-distance phone number when you dial out.  Scroll to the phone number,

press Return, press the End key to move the cursor to the end of the phone

number, type in your account number, and press Return again.  Insert a

comma to generate a pause if you need one.  The default pause in virtually

all modems is 2 seconds for each comma.  (IMPORTANT:  Please just modify

the phone numbers that are there, and NEVER configure to dial our phone

number.  The program will refuse to run if you put our number in it, and we

don't have an atomic clock anyway.)

Editing works the same way for the user's modem initialization string.

It's especially important to study the help screen for that one, because it

has some crucial information about what to use with different types of

modems.

Configuration for the type of clock in your computer is a special topic and

is discussed in the next section.

- 4 -

Be sure that no configuration fields are left blank and none contains the

word "(edit)".  The word "(edit)" appears on a couple of the editable

configuration items when you press the right or left arrow key enough

times.  When you see it, you are supposed to get into editing mode and

replace it with actual configuration data.  Rest assured that the program

will not work properly if configuration fields are left blank or contain

"(edit)".

When you've checked or changed all the configuration items and are

satisified that everything is correct, press F10 to save the configuration

to TIMESET's source file on disk.  As soon as the configuration is saved,

the program jumps into operating mode, reads its new configuration, and is

ready to call either of the time services.

CONFIGURING TIMESET FOR HARDWARE CLOCKS

Every computer comes equipped with at least one clock, called the DOS

clock/calendar, which can be read or set with the TIME and DATE commands in

DOS or by a program such as TIMESET.  Its major limitation is that it's

volatile; that is, the time and date settings are lost whenever the

computer is turned off, and both have to be reset each time the computer is

restarted.

Because of this volatility, battery-powered clocks have been developed to

maintain the current time and date while the computer is off and pass them

to the DOS clock/calendar when the computer is turned back on.  There are

three kinds of nonvolatile clocks:  (1) the CMOS clock found on the

motherboards of AT and later computers, (2) add-on clocks that plug into an

expansion slot in the computer, and (3) so-called slotless clocks that plug

into a socket under the ROM chip or some other socketed chip.  TIMESET can

work with nearly all of the first and second types, but not with the third.

"Slotless Clocks" and Tandy's SmartClock

TIMESET can work with CMOS clocks and most add-on clock boards.  However,

it will not work directly with Tandy's SmartClock nor with slotless clocks.

With those clocks you'll have to configure to set the DOS clock/calendar

with TIMESET then later on use your own clock's software to reset the add-

on clock.

CMOS Clocks

Depending on the version of DOS that is installed in a computer, a CMOS

clock may need special software to be reset or to make it update the DOS

clock/calendar.  With DOS 3.3 or later, you need no special software,

because those DOS versions automatically keep the CMOS clock and the DOS

clock/calendar synchronized, so changes made in the DOS clock/calendar are

simultaneously made in the CMOS clock as well.  That automatic

synchronization doesn't occur with DOS versions earlier than 3.3, however.

- 5 -

Therefore, if your computer has a CMOS clock, the version of DOS you're

using will determine how to configure TIMESET 6.0 for clock type.  If it's

DOS 3.3 or later, you should configure TIMESET to set only the DOS

clock/calendar, because the operating system will automatically update the

CMOS clock when TIMESET changes the DOS time and date.  If you have an

earlier version of DOS, you will have to configure TIMESET 6.0 so it can

directly set both the DOS clock/calendar and the CMOS clock, thus

eliminating the need to run DOS's SETUP program.  You can select the

correct option by using the right or left arrow key in configuration mode.

If you need help, press F1 while that item is highlighted.

Expansion Slot Add-On Clocks

Add-on clocks that plug into expansion slots present a mixed situation.

Some require you to run a special program each time you want to update the

DOS clock/calendar from the add-on clock or update the add-on clock from

the DOS clock/calendar.  Others come with a memory-resident program or

driver that takes care of these time adjustments between the two clocks

automatically, making them behave very much like the CMOS clock under DOS

3.3 or higher.  Configuring TIMESET 6.0 for these add-on clocks therefore

depends on the kind of software that comes with the clock.

Better known brands of add-on clocks these days come with memory-resident

software to keep the add-on clock and the DOS clock/calendar synchronized

automatically.  An example of such a program is ASTCLOCK, for managing the

clock on AST's Six-Pack multifunction board.  Running ASTCLOCK with the /R

switch on the command line (ASTCLOCK /R) makes it memory-resident so it can

automatically keep the two clocks synchronized.  If your add-on clock came

with a memory-resident driver program or a driver that goes in your

CONFIG.SYS file, install that driver and configure TIMESET to set only the

DOS clock/calendar.  Your driver software will take care of resetting the

add-on clock when TIMESET sets the time and date in the DOS clock/calendar.

Using CLKDRV10.SYS with Board Clocks

There are a number of slot-type add-on clocks that do not come with memory-

resident software and therefore can't pass time and date changes back and

forth between the two clocks automatically.  These clocks, with mail-order

prices ranging from $7.50 to $15, have obscure brand names and often no

brand name at all.  Despite their low cost and generic nature they are

surprisingly good, often more accurate and stable than the CMOS clocks that

come with expensive computers.  For example, mine, which is installed in a

8086 computer, loses about 1.5 seconds a day, whereas the CMOS clock in my

wife's 386sx gains about 25 seconds a day.

The major limitation of these cheap add-ons is lack of memory-resident

software to allow automatic movement of time and date between the two

clocks.  Instead they require you to run a special program to update the

add-on clock each time you change the time or date in DOS (e.g., SETCLOCK),

- 6 -

or to update the DOS clock/calendar from the add-on clock (e.g., GETCLOCK).

With some of these clocks, both functions are handled by a single program,

with the desired function chosen with a command line switch.  Having to go

into DOS to run external programs like these is utterly impractical for a

program like TIMESET, which requires timesetting to occur in both clocks at

virtually the same instant for maximum accuracy.  Furthermore, some of them

insist on prompting for the time, which of course means the time will be

wrong no matter how fast a typist you are.

For people with these obscure brands and that clumsy kind of software, the

TIMESET 6.0 package includes a small driver called CLKDRV10.SYS, written by

Tom Strickland of Alexandria, Virginia.  To use it, put the driver in your

boot directory and the following line in your CONFIG.SYS file:

device=clkdrv10.sys

Loading CLKDRV10.SYS in effect erases the barrier between your DOS

clock/calendar and your add-on clock and eliminates the need for any other

software to manage the two clocks and keep them synchronized.  Thus, if you

change time or date in DOS, CLKDRV10.SYS sees to it that the add-on clock

gets the same change, and if you ask for time or date in DOS or a program,

CLKDRV10.SYS automatically updates the DOS clock/calendar from the add-on

clock an instant before DOS displays the time or date.  With CLKDRV10.SYS

installed, you must configure TIMESET to set only the DOS clock/calendar,

and let CLKDRV10.SYS take care of updating the add-on clock.

CLKDRV10.SYS is designed to work with board-type add-on clocks that have a

base clock address of 240h, 2C0h, or 340h, and it automatically checks to

see which of those ports is being used by your clock.  Nearly all expansion

slot add-on clocks use one of those three base addresses, and some of them

allow you to choose between at least two of them by means of a jumper on

the clock board.  If your clock does not use one of these three port

addresses (which is possible but infrequent), CLKDRV10.SYS will not load

and you will have to be content with letting TIMESET adjust only the DOS

clock/calendar.

If you install CLKDRV10.SYS, you should put your clock's regular software

aside, because CLKDRV10.SYS has its own method of managing the add-on

clock.  You won't need your original software anyway, because CLKDRV10.SYS

makes your add-on clock directly accessible through the TIME and DATE

commands in DOS or from any program.  CLKDRV10.SYS is mainly for people

whose add-on clock did not come with its own memory-resident driver

program.  If your add-on clock came with its own memory-resident software,

use it rather than CLKDRV10.SYS.

It's important to know when not to use CLKDRV10.SYS.  You should NOT use it

when: (1) your computer has a CMOS clock, or (2) you have an add-on clock

board that came with its own memory resident software to accomplish what

CLKDRV10.SYS does, or (3) you have only the DOS clock/calendar and no

battery-powered clock board at all.  Under no circumstances should you load

both CLKDRV10.SYS and another driver that accomplishes the same thing --

the two drivers will get into a contest that could paralyze the system.

- 7 -

Also, CLKDRV10.SYS will be of no help if you have Tandy's SmartClock or a

"slotless" clock.  (The manufacturers of those clocks should be pressured

to develop their own memory-resident drivers for them.)

RUNNING THE PROGRAM

Options at the Main Menu

Dialing

If you configured for interactive mode, dialing starts when you press W,

Alt-W, B, or Alt-B.  W stands for Washington (Naval Observatory) and B

stands for Boulder (NIST).  Pressing W or B alone gets the time without any

adjustment for measured line delay.  In combination with the ALT key,

pressing W or B gets the time with a line delay adjustment from Washington

(Alt-W) or Boulder (Alt-B).  Line delay measurements are discussed in more

detail later on.

If you configured the program to run in noninteractive mode (no prompts),

you won't have to press any keys; TIMESET 6.0 will load, immediately dial

the time service you chose, set the time, and exit to DOS.  The choice of

interactive or noninteractive mode is yours.  If you plan to have TIMESET

in your AUTOEXEC file so it can update you computer clock during the boot-

up process, it's best to configure for noninteractive mode so timesetting

will be automatic and booting can continue after the clock has been set.

TIMESET skips the main menu when it's in noninteractive mode and there are

no pauses for input.

Standard and Daylight Time

One way to switch between daylight and standard time is to press F2 at the

main menu to get into configuration mode, scroll down to the time season

line, toggle daylight or standard with the right or left arrow key, and

press F10 to save the change.  You'll need to do it that way during initial

configuration since you are automatically put into configuration mode when

you run a fresh copy of TIMESET 6.0, and the program won't run until it's

fully configured.  However, after you have configured the program and it's

time to reconfigure for daylight or standard time there is a very simple

way to do it from the main menu:  just press F7.  That will not only change

change the configuration, it will simultaneously reset your computer clock

for the new season.  You should do that reconfiguration before dialing one

of the time services on the day the time season changes.

Monitor Color

Although you can specify color or monochrome display during configuration,

a function key toggle is also available at the main menu for controlling

monitor color.  Pressing F8 at the main menu switches the display between

color and monochrome.  You may need to force monochrome display this way if

- 8 -

you run TIMESET 6.0 on a laptop computer that shows colors as shades of

"grey"; some colors have poor contrast on certain laptops.

Pulse Generation

F9 is a special key that will probably not be needed by many people.

Pressing it at the main menu allows a single pulse to be generated at pin 1

of the parallel printer port at the instant of timesetting.  The pulse can

be used to synchronize external devices, which could include specialized

(and expensive) clocks.  It can also be used for calibrating instruments.

According to the NIST, its time mark typically has repeatability of about 1

ms from day to day when obtained with line delay correction.  This allows

the delay-corrected pulse generated by TIMESET to be used in combination

with a digital counter to check or calibrate an oscillator with high

accuracy.  The uncertainty would be on the order of 1 part in 100 million

for oscillation counts at 1/second made a day apart (0.001 second

repeatability divided by 86,400 seconds/day).

For reference, here are the characteristics of the pulse:

Change:  ~5 volts

Direction:  from ~+5 volts to 0 volts (relative to pin 25)

Duration:  ~55 ms (controlled by the system timer)

Depending on the external device's requirements, it may be necessary to

invert the pulse or modify it in other ways with appropriate electronic

circuitry (which is beyond the scope of this manual).

A companion program, PULSEGEN, is available for $15 from Life Sciences

Software for generating continuous pulsing at the parallel port at 1/min,

1/sec, 10/sec, or 100/sec with any computer, and up to 1000/sec with

computers equipped with slot-type add-on clocks using base I/O port

addresses of 240h, 2C0h, 340h, or 3C0h and capable of millisecond

resolution (probably all such clocks are, although their port for

thousandths is usually not used since the DOS clock resolution is no better

than 0.01 sec).  The pulse widths produced by PULSEGEN depend on CPU speed

and are in the order of microseconds; i.e., no intentional delay is

inserted between pulse on and pulse off.  The running time display is to

three decimal places and is written directly to the video buffer for

maximum speed.  If the computer clock has been freshly set with TIMESET,

the pulses, which can be used for driving external devices continuously,

will occur approximately in real time, subject to clock drift.

Pulses should not be generated at the parallel port if a printer is

attached.

- 9 -

Changing the Configuration

If TIMESET 6.0 is configured for interactive operation and you need to

change or check the configuration, press F2 at the main menu to get the

configuration screen.  If you make any changes, be sure to press F10 to

save them, otherwise you can press F2 to go back to the program without

making any changes.  Getting to the configuration screen can require an

extra step if you have already configured TIMESET to run in automatic mode,

because it then bypasses the main menu and starts dialing as soon as you

run it.  In that case you can press the Backspace key to interrupt dialing

and go to the main menu, where you can press F2 (there's plenty of time to

do that before the call goes through).  A better alternative is to load

TIMESET 6.0 with /C on the command line, which forces it to come up in

configuration mode so you can make your changes.

Additional Help

Pressing F1 at the main menu brings a help screen that summarizes your

keyboard options within the program.  There are also some command line

options for TIMESET 6.0, including options for temporarily overriding some

of the configurations or delaying dialing until a specified time.  You can

see these command line options by typing TIMESET ? at the DOS prompt.

THE TIME DATA STRINGS

The data strings from the Naval Observatory and the National Institute of

Standards and Technology have a few similarities and several differences.

The pattern for both services is a string of data that announces the next

second, followed by a brief pause, then a character that marks the second.

This goes on endlessly, every second, 24 hours a day, year round.  When you

are connected to either service with TIMESET 6.0 these data strings will be

displayed on your screen as they are received for as long as it takes to

get a clean data string to set your computer clock.  With clean lines and

no line delay measurement, that's about 2 or 3 seconds with the Naval

Observatory and about 5 or 6 seconds with the NIST.  If you request line

delay measurement, it takes about 11 seconds online with the NIST and about

21 seconds with the Naval Observatory.  These times will increase if there

is a lot of noise on the phone lines, because TIMESET will reject data

strings that are corrupted by noise and don't match the required pattern.

Both services limit your time online.  The Naval Observatory gives you a

maximum of 60 seconds.  The NIST gives you up to 55 seconds but will reduce

that to 15 seconds if the service is in heavy use.  Except for extremely

noisy lines (which may indicate a problem with your own phone system),

these times are more than adequate for TIMESET 6.0.

- 10 -

The Naval Observatory's Data String

The pattern of the Naval Observatory's time data string is:

JJJJJ DDD HHMMSS UTC (pause) *

JJJJJ is the last five digits of the julian day, which is simply

a count of the number of days since January 1, 4713 B.C.  The

first two digits, 24, are left out because they will not change

for a long time to come (140 years or so) and can be assumed in

calculations.  TIMESET uses the julian day to calculate the

current month, day of the month, and year.

DDD is the day of the year (number of days since December 31 of

the previous year).  It is only displayed by TIMESET, at the

final screen.

HHMMSS is what the hours, minutes, and seconds will be when the

time mark -- the asterisk (*) -- is sent.

UTC stands for Universal Time Coordinated, and it means that the

values for julian day, day of the year, and the time all apply to

0 degrees longitude, the longitude of the Royal Observatory at

Greenwich, England.  TIMESET corrects these values for your time

zone and season before setting your computer clock (unless you

want your computer on universal time, which is desired by some in

the aviation industry and the military, for example, and have

configured TIMESET for it).

The NIST's Data String

The NIST time data string is more complex:

JJJJJ Yr-Mo-Da HH:MM:SS Sn L UT1 msADV UTC(NIST) (pause) * (or #)

Again, UTC means the time and date values all pertain to Greenwich,

England.

JJJJJ means exactly what it means in the Naval Observatory

string.  Unlike the Naval Observatory, the NIST does not provide

the day of the year, but TIMESET 6.0 calculates it from this

number and displays it on the final screen.

Yr-Mo-Da means current year, month, and day of the year.

HH:MM:SS means hours, minutes, and seconds.

Sn is a two-digit number that indicates whether the continental

United States is on standard or daylight saving time or is

approaching daylight or standard time.  Except for a 48-day

period before a time season change, the number is either 00

- 11 -

(standard time) or 50 (daylight time).  In the fall, the number

begins to decrement daily, starting 48 days before standard time

officially begins in late October, when it reaches 00.  In the

spring, the number begins to increment daily from 00, starting 48

days before daylight saving time officially begins, when it

reaches 50.  During the periods when Sn is either incrementing or

decrementing, TIMESET 6.0 prints a special message on its final

screen announcing the number of days (including the current day)

before the time season is due to change in the continental United

States (the days are based on UTC).  At all other times it

displays the current time season in the continental U.S. on the

final screen.

L is a flag that indicates whether a leap second will be added or

subtracted at midnight on the last day of the current UTC month

to compensate for irregularity in the earth's rotation.  If a

leap second is to be added, the number is 1.  If L is 2 it means

that a second will be subtracted at midnight on the last day of

the current UTC month, but the NIST says a negative leap second

is not likely to occur any time soon.  The L flag is on for the

entire month before a second is added or subtracted, which will

usually be in June or December.  Most of the time the L flag is

set at 0, meaning no change.  The flag is merely an announcement

and is not needed by TIMESET 6.0.

UT1 is a correction factor for converting UTC to an older form of

universal time that is still used in navigation.  It is the

tenths of a second to be added or subtracted from UTC and is

preceded by the appropriate sign.  It is only displayed by

TIMESET 6.0.

msADV is the NIST's line delay correction and is discussed as a

separate topic in the next section.

LINE DELAY MEASUREMENTS

It is physically impossible for the time mark character at the end of the

data string to arrive at the same instant it leaves the time source.  There

is always a delay, and it is the sum of three components:  (1) the time it

takes for data bits moving at the speed of light to travel the distance

between the time source and your computer, plus (2) the time it takes for

the time source's modem to serialize and send the time mark character, plus

(3) the time it takes your modem to process the incoming data and feed it

to your computer.  With land line communication in the continental United

States, the time the signal spends traveling between the modems is almost

negligible compared to the processing delay that is inherent in modems.

With a satellite link, however, the time the signal spends traveling

becomes very significant and accounts for most of the lag.

- 12 -

Both the NIST and the U.S. Naval Observatory are equipped for measurement

of line delay, but each uses a different method.  TIMESET 6.0 can handle

either method.  When TIMESET 6.0 asks for line delay measurement from the

NIST, the NIST itself measures the delay and advances the departure of the

time mark to compensate so the mark arrives on time.  However, with the

Naval Observatory, TIMESET 6.0 itself measures the line delay by means of a

remote digital loopback test, in which the program sends a stream of

characters to the Naval Observatory and measures the time for the Naval

Observatory's modem to echo them back.  It then captures a time data string

and adds the measured one-way delay to the time as a correction before

setting your computer's clock.

Notice the essential difference in methodology here:  delay measurements

with the Naval Observatory are made by TIMESET; delay measurements with the

NIST are made by the NIST.

Because of limitations in standard computers, the line delay measurements

the NIST makes are inherently more precise than the ones TIMESET 6.0 makes

with the Naval Observatory.  The NIST's equipment can measure line delay to

the tenth of a millisecond.  In contrast, TIMESET must round the line delay

it measures with the Naval Observatory to the nearest 0.01 sec since that's

the smallest unit of time that a standard computer can accept as a

correction.  This is a 100-fold difference in precision.  It's important to

realize, however, that this difference in precision has absolutely no

effect on the accuracy of timesetting in your computer.  Precision and

accuracy are not the same thing.  Furthermore, the "granularity" in

computer clocks, which is based on the ticking rate of the system timer

(18.2 times per second), limits the accuracy with which they can be set to

within about 0.055 sec of true time, which means that timesetting accuracy

ends up the same with either service.

(Delay correction and accuracy are discussed in more detail later.)

Line Delay Correction by NIST

msADV in the NIST's data string is the milliseconds (ms) that the departure

of the time mark was advanced to compensate for line delays.  If no line

delay measurement is requested, the NIST routinely advances the departure

of the mark by 45 ms (0.045 sec), which is displayed in the data string as

045.0.  The standard 45 ms is an estimated value for the United States that

includes the 8 ms it takes to send all the bits of the time mark character

at 1200 baud, plus an assumed 7 ms for travel from the NIST to the average

user in the United States, plus 30 ms to compensate for the inherent delay

in 1200 baud modems (the modem delay is an average value for several modem

brands).

If an actual line delay measurement is requested by initiating the call

with Alt-B, the NIST itself calculates the delay, then advances the

departure of the time mark to compensate for the measured one-way lag so

the time mark can arrive at your computer at the right instant.  As soon as

the NIST has measured and adjusted for line delay, the time mark changes

- 13 -

from an asterisk to a pound sign (#).  TIMESET watches for the pound sign

and sets the computer clock the instant it is received.

The NIST says that if your modem has the same internal delay as theirs, you

should receive the # time mark within +-2 ms of the correct time.  If it

does not have the same internal delay, the mark should arrive within +-10

ms of the correct time, with different modem brands varying within that

range.  Repeatability of line delay correction is excellent, typically

within 1 ms from day to day according to the NIST.  Accuracy of setting the

time in your computer is another matter, however, and is discussed

separately below.

The measured line delay is likely to be slightly larger than the estimated

45 ms that the NIST routinely uses when you don't request an actual

measurement and are using land lines, and it's much larger if the

communication is by satellite.  It will be influenced by the speed of your

computer's CPU in echoing characters to NIST as well as by distance and the

characteristics of your modem.  With a Hayes Smartmodem 2400 and a computer

running at 8 Mhz, the delay measured between the Seattle area and Boulder,

Colorado is always around 62 ms (0.062 second).  Beta testers using TIMESET

6.0 in New Jersey, Maryland, and Illinois with 20-25 Mhz 386 computers and

different modem brands have reported delays around 54 ms.  A tester in

Alaska, with a 386 computer and a U.S. Robotics modem reported a delay of

just over .3 second, a magnitude of delay that indicates a satellite link.

Our tests show that the measured line delay is also affected by the choice

of long distance carrier.  The delay is consistently greater with MCI than

with AT&T -- about 62 and 54 ms respectively calling from the Seattle area

with a land line link in each case.  The accuracy of setting is the same in

either case, however, since the NIST adjusts for the lag it measures.

In the usual, bidirectional, satellite link, the one-way signal delay is in

the range of 260 to 300 ms (.26 to .3 seconds).  The communications

satellites are parked in stationary orbits about 26,000 miles from earth,

which requires the NIST's time mark to travel 52,000 miles or more to reach

the caller's computer.  This alone takes over a quarter of a second with

the signal traveling at the speed of light (186,000 miles/sec).  Modem

processing adds further delay.

There is one situation where the NIST is unable to report a line delay

measurement.  That is when the path is by satellite (a long path) in one

direction and by land line (a shorter path) in the other.  In that

situation, which the NIST detects by a delay measurement of 90-260 ms, it

is impossible to know whether it's the long path or the shorter one that is

carrying the time signal to the user, so a delay adjustment can't be

applied; the NIST time mark remains an asterisk and the standard 45 ms

advance is used.  According to the NIST, a split-path transmission like

this is rarely used by telephone services.

- 14 -

Line Delay Correction with the Naval Observatory

If you dial the Naval Observatory without asking for line delay

measurement, TIMESET 6.0 will routinely add 50 ms (0.05 sec) to the time it

feeds to the computer clock.  This value is close to the standard 0.045 sec

time mark advance the NIST makes when no actual line delay measurement is

requested, has the same purpose, and includes the same estimated delay

elements (see first paragraph in the preceding section).  Why is the

estimate for the Naval Observatory 0.05 second instead of 0.045 second?

The answer is that it's impossible to set a computer clock to the third

decimal place; 0.045 sec must therefore be rounded up to 0.05 sec.  The

practical effect of this rounding up is nil; if you dial the two services

alternately and in quick succession without requesting line delay

measurements, the before/after time readings on the final screen will

usually agree within 0.01 sec. each time.

Actually, it makes sense to have the default line delay correction slightly

larger for the Naval Observatory since Washington, D.C., on the east coast,

is farther away from the statistical population center of the nation than

more centrally located Boulder, Colorado.  The default correction will

therefore be about right most of the time for most users in the United

States.

An actual line delay measurement for Naval Observatory data is made by

means of a remote digital loopback test, which not all modems can do.  Even

if your modem can do remote digital loopback testing, if it doesn't use the

&T6 command to initiate the test and the &T0 command to terminate it, you

will not be able to make this measurement with TIMESET.  An example of a

modem that can do remote digital loopback but uses commands other than &T6

and &T0 is the U.S. Robotics HST.  There is no reliable way for a program

to detect a modem's ability to handle &T6 and &T0, so it's up to you to

check your modem manual to see if you can do it.  If you ask for remote

digital loopback and your modem can't do it, you'll just sit there online

wasting money and accomplishing nothing until the Naval Observatory hangs

up on you or until you press the backspace key to hang up and go back to

the main menu.  People with such modems who need line delay measurement

should use the NIST's service.

In a remote digital loopback test, TIMESET instructs the local modem to

send a request for the test to the Naval Observatory's modem.  This

conditions the Naval Observatory's modem to echo back characters that

TIMESET sends it.  TIMESET 6.0 calculates the one-way line delay by

measuring the time it takes for 20 characters to travel to the Naval

Observatory and return.  When 20 characters have been sent and echoed back,

TIMESET turns off the remote digital loopback test, calculates the average

lag per character, reverts to normal operation, and grabs one of the Naval

Observatory's regular time data strings.  The calculated one-way line delay

per character, rounded to the nearest 0.01 sec, is added to the zone- and

season-adjusted time just before it is sent to your computer's clock.

- 15 -

When Should Line Delay Measurement Be Used?

The idea of being able to measure and correct for line delay so you can set

your computer clock with maximum accuracy is fascinating to many people.

It intrigues us, too, which is why we worked very hard to incorporate that

feature in TIMESET for both time services.  However, it has the potential

of being overused or used unnecessarily.

A major purpose for measured line delay adjustment is for satellite

communications, where line delay is a significant factor.  For ordinary

purposes within the continental United States, where telephone

communications are virtually always by land lines, the default standard

estimates should normally be adequate, because they are so close to what

can be obtained by actual measurement.  For example, TIMESET's built-in

default estimate of line delay for the Naval Observatory's signal is 0.05

seconds, whereas the measured delay from Washington, D.C. to Seattle,

Washington is typically around 0.0700 -- a difference of 0.02 sec., and

usually less.  Similarly, the standard estimated line delay the NIST

provides is 0.045 second, whereas the NIST's measured delay between

Boulder, Colorado and Seattle is typically around 0.062 sec --less than

0.02 second difference.

These are very small discrepancies that are so close to the smallest time

unit that can be read with a computer clock (0.01 sec) that they are hardly

worth worrying about.  If you want to see how small 0.02 second is, try

reading numbers in the second decimal place in one of TIMESET's running

time displays.  You can't.  They change so fast that they blend into each

other.  We included the second decimal place in those displays just to

remind you how unimportant 0.02 sec is for most purposes and to persuade

you that there's no sense in going overboard with line delay measurements.

Why go to all that time and trouble for a a couple hundredths of a second

more accuracy when your computer clock could drift that much in the next

several minutes?  Furthermore, computer clock updating can take place only

when the system timer ticks, every 0.055 sec, which means you can only be

confident that your clock is set somewhere within 0.055 second of true

time, a range of error nearly three times larger than that extra 0.02

second of accuracy you're striving for.  Clearly, measured line delay is

useful only with satellite communications, where line delays are

substantial, or for calibrating external equipment in real time.

Another thing to consider with line delay measurement is the extra time on

line that it requires, as indicated in the following table:

Connect time typically required (seconds)

------------------------------------------------------------

Delay measurement              No delay measurement

-----------------------------    ---------------------------

Naval Observatory   21                                 2

NIST                12                                 6

- 16 -

The size of these connect time differences suggests that if a lot of people

get in the habit of doing pointless line delay measurements they will

effectively reduce the capacity of these services.  The result for

everybody could be a lot of frustrating busy signals.

Be scientific.  Use it sensibly.

A Note About MNP Modems

TIMESET 6.0 will work if you are using an MNP modem and it's initially set

for MNP error checking.  However, MNP isn't needed because neither the NIST

nor the Naval Observatory uses it.  Furthermore, tests show that MNP action

can increase the line delay that is measured by the NIST, since it delays

the return of echoed characters at the same time the NIST is measuring the

echo time to determine one-way line delay.  This should not affect

accuracy, because the delay is real and the NIST adjusts the departure of

the time-mark character to compensate for it if you request line delay

measurement.  Still it's something you need to know about.  If you don't

request line delay measurement and use MNP, the nominal 45 ms standard

advance used by the NIST, or TIMESET's 50 ms standard adjustment for Naval

Observatory signals, will provide even less compensation for delay than

they otherwise would, because of the slowing action of MNP.

MNP action could also require you to be online a few seconds longer than

with an ordinary modem or an MNP modem with MNP turned off, because the

first thing an MNP modem does is check for MNP action and MNP level in the

remote modem.  If it doesn't detect MNP in the remote modem, it reverts to

standard operation (provided it's configured to do so, which it should be

if you are calling one of the time services).  All that can take a few

seconds, and during that time data strings being transmitted are not being

seen by TIMESET.  You are wasting time on line, in other words.  If you

happened to call NIST at a time of day when there was a lot of phone

traffic, a few seconds worth of wasted data strings could make a

difference, especially if there was also line noise to corrupt data strings

and cause their rejection by TIMESET.  Since the NIST may limit your

connection time to 15 seconds during busy periods, you could be cut off

before your computer clock gets set.  For this reason you might need to

consider turning off MNP when doing line delay measurements.

THE FINAL DISPLAY

As soon as the time is received and the computer clock is set, TIMESET

jumps to its final screen, which displays the NIST or Naval Observatory

data string that was used to set the computer's clock, along with extracted

information about UTC time and date, daylight/standard time announcements

if the NIST was used, a table showing the computer's time and date before

and after the call, the estimated or measured line delay correction, and

the time it took TIMESET to process the data and set the clock (internal

delay).

- 17 -

The internal delay is an actual self-measurement by TIMESET of the time it

took to process the string of data it received.  There's a lot to do with

that string -- parse it, convert its components to numeric form, calculate

the date from the julian day, adjust for local time zone and season, create

new strings from numbers for summarizing results on the screen, etc.

Practically all the internal delay is due to string manipulation, one of

the slowest processes in a computer.  With our 8086 computer the internal

delay is always about 0.01 second, but it might well be too small to

measure with a very high-speed computer.  The value for internal delay is

added to the time that is sent to the computer clock.  If the call was to

the Naval Observatory, the estimated or measured line delay is also added

to the time that was sent to the computer clock.  With a call to the NIST,

only the internal delay will be added (line delay will not be included,

because the NIST compensated for it by advancing the time mark's departure

so it could arrive on time).

Current system time to two decimal places is shown in a running display in

the middle of the final screen if TIMESET 6.0 is configured for interactive

mode.  The main menu screen's large clock display is available at this

point by pressing the backspace key to go back to the main menu.

A FURTHER NOTE ABOUT ACCURACY

It's useful to remember that even though TIMESET 6.0 links you to the most

accurate clocks in the world, your computer's clock can never be set to

their accuracy.  The main reason is that a computer clock is not

continuously receptive to timesetting.  Because of the way standard

computers are designed, their clocks "tick" (advance) about 18.2 times a

second, with variations among computers, which means they are updated in

approximately 0.0549-second increments (1 divided by 18.2), although they

can display time to 0.01 second.  So even if a time mark arrives within

+-2 ms of true time, this "granularity" in computer clocks means you can

only be sure that your clock has been set within about 55 ms (55/1000 sec)

of true time, which is very accurate indeed but not perfect.

Repeatability of timesetting with TIMESET 6.0 is very good.  In tests where

one service or the other is called a few times in quick succession, before

the computer clock has time to drift to any measurable extent, the

"Computer time was:" item on the final screen is almost always within 0.01

second of what it was after the previous call.  Usually it's identical.

It's important to remember, however, that you shouldn't expect your

computer clock to stay highly accurate very long.  No computer clock does.

(If your computer time is wildly unstable and you have a battery-powered

clock in it, it could mean the battery needs replacing.)  Get to know your

computer clock's stability by running TIMESET a few times over a period of

days.  How often you need to run it thereafter depends on how stable your

clock is and the degree of inaccuracy you can tolerate.

- 18 -

If you are a scientist or engineer and need accurate time for physical

measurements, you already know you'd need to run it just before a

procedure.  Same thing if you are part of a far-flung organization and need

to have computers in the outposts synchronized with a reliable and mutually

accessible standard (but don't forget to pay for each copy of TIMESET).  If

you are part of a radio, TV, or cable network you might need to run it at

least a couple times a day so you won't switch into the network in the

middle of somebody's sentence.  If you are satisfied to have your clock

accurate within a few seconds, a run every day or two should be sufficient.

If accuracy within 1 minute is enough, you probably need to run it only

once every few days.  If you're satisfied to be within 5 or 10 minutes, you

can probably wait weeks, but you might want to consider just checking the

clock on the wall and setting your computer clock manually.  If clock

accuracy doesn't matter to you, then of course you won't need TIMESET at

all.  We definitely recommend that these valuable time services be used no

more than necessary, and we urge people not to abuse them.

TIMESET 6.0 AND THE WORDPERFECT LIBRARY

TIMESET 6.0 contains assembly language routines for automatic detection and

full compatibility with the WordPerfect Library, an excellent program for

task-switching made by the WordPerfect Corporation.  It behaves exactly

like WPCorp's own programs when managed by the Library, allowing you to:

(1) load TIMESET memory-resident or remove it from memory, both invisibly;

(2) directly access it from other programs in the Library menu by pressing

hot keys; and (3) directly access the other programs from TIMESET by

pressing hot keys.  Depending on how you load the Library, it swaps TIMESET

with the other programs by using either expanded memory (EMS) or a

temporary hard or RAM disk file as the storage area for the program's

memory image.  TIMESET 6.0 takes about 136 kbytes of EMS or disk memory

under the Library and about 135 kbytes under DOS.

- 19 -

PROGRAMMING NOTE AND ACKNOWLEDGEMENTS

November 19, 1990

TIMESET 6.0 was written with Microsoft's BASIC Professional Development

System version 7.0 and contains scores of linked commercial assembly

routines as well as several special purpose assembly routines I developed

when no commercial alternatives were available.

It's doubtful that using another higher-level language to write the program

would have improved anything, and I thank the folks at MicroHelp Inc.,

Crescent Software, and Microsoft for transforming BASIC into a truly

professional language.

My special thanks go to a group of special people who tested TIMESET 6.0

for me during its development and made many helpful suggestions:

Tom Strickland, Virginia

Bob Germer, New Jersey

Bruce Felstein, New Jersey

Jim Luhman, Illinois

Stu Rothman, Alaska

Tom Takesian, Maryland

Julia Petrakis (my wife), Washington

The CLKDRV10.SYS driver for expansion slot add-on clocks was created by Tom

Strickland of Alexandria, Virginia and fills an important void.  It has

been part of the TIMESET package for over two years and is an outstanding

piece of work.  I'm very grateful to Tom for letting me include it.

Pete Petrakis, Life Sciences Software

Stanwood, Washington (Compuserve 76555,1175)

- 20 -

CUSTOM PROGRAMMING SERVICES AVAILABLE

FROM LIFE SCIENCES SOFTWARE

If you need custom programming, write or phone Life Sciences Software, tell

us what you want, and find out if we can help you.  The chances are good

that we can, and with the same care and experience that has gone into

TIMESET and a lot of other very well received programs we've written.  But

you won't know unless you ask.  Our rates are competitive.

You can telephone us at 206-337-7328 between 8 a.m. and 5:30 p.m. Pacific

time, Monday through Friday (the phone number will change to 206-387-9788

after January 1, 1991).

Our address is:

Life Sciences Software

8925 271st N.W., Suite 112

P.O. Box 1560

Stanwood, Washington 98292

- 21 -

DISCLAIMER

TIMESET is a mature program that has been thoroughly tested through several

revisions since its inception in early 1987.  Tremendous effort has been

expended to make it work properly with as many systems as possible.  This

includes the bewildering variety of modems that exist, some of which may be

described charitably as balky and idiosyncratic.  The number of modem

brands that TIMESET cannot work with has declined with each refinement over

the years, and dramatically since the release of version 5.3 in the fall of

1989.  Only a few exceptionally stubborn ones have proved unyielding so

far.  It is our hope that version 6.0 will work for everybody no matter

what kind of equipment they have, but of course we cannot guarantee it.

The virtue of shareware is that you can determine a program's usefulness to

you before spending any money on it.  Nor can we accept any responsibility

or liability for any harm the program might cause to your system or your

data.  We have never heard of any bad effects, have never experienced any

ourselves, and cannot imagine how there could be any, but, as with all

programs, the user must assume any risk.

- 22 -

YOUR SUPPORT IS VERY IMPORTANT

We hope you like Professional TIMESET 6.0 and find it useful.  Please

register and pay for it if you do.  It has taken a lot of hard work and

study; many, many hours (surely thousands since the first version in 1987);

and considerable expense to develop TIMESET into the highly sophisticated

program it is and the best of its kind.  If it's useful to you or your

organization, it merits your financial support.

It's also advantageous to you to register TIMESET.  Registration brings you

fresh copies of TIMESET and its supporting programs (including any interim

fixes before the next major upgrade), a printed manual, free technical

support, and a 20 percent discount on the next major revision.  Handling

and mailing are included in the $35 per unit registration price.  Please

fill out the form on the next page and mail it to us with payment, or

register by modem with TSREGSTR and be invoiced if you prefer.  (Note:

Registered users of versions 5.0-5.3 will automatically receive their

copies of version 6.0 as promised and need not register again.)

By the way, we always like to get reports from TIMESET users -- favorable

or unfavorable.  They are what have driven the program's evolution from the

beginning.

- 23 -

TIMESET 6 MAIL REGISTRATION FORM

For Individuals and Organizations

Notice to companies and other organizations:  Registration and payment is

required for each copy of TIMESET in use within an organization.  The price

is $35 per copy for up to six copies.  Please call Life Sciences Software

at 206-337-7328 for discount information on more than six copies (the phone

number will be 206-387-9788 after January 1, 1991).

Notice to all:  If you prefer to register TIMESET by modem and be invoiced

for payment, use the TSREGSTR program to send the registration information

to our electronic mailbox.  The information TSREGSTR will prompt you for

and send to us is essentially what you see below.

TO:  LIFE SCIENCES SOFTWARE

8925 271st, N.W., SUITE 112

STANWOOD, WASHINGTON 98292

I wish to become a registered user of TIMESET.

My name: _____________________________________________________________

Organization name (if any): __________________________________________

Street address: ______________________________________________________

Additional address: __________________________________________________

City: _______________________  State: __________________ ZIP: ________

Phone number: ___ ___ ____

Number of copies being registered: ___

PAYMENT:

Note:  As a licensed business in the State of Washington we are required to

charge residents of the state an additional 8 percent for sales tax.

___ I am enclosing payment for ___ copy(ies) of TIMESET at $35.00

per copy (Washington residents please add 8 percent for sales

tax).  Total enclosed:  U.S. $ ________

Please make your check or money order payable to Life Sciences Software.

Thank you very much.

- 24 -