This  file  supplements the information

in the  manual  about  using  the  PCjr

Booster.

Contents:

MEMORY.SYS /s: and /v: Switches

Running Flight Simulator

---

MEMORY.SYS /s: and /v: Switches

The MEMORY.SYS /s:  and /v:  switches

determine how PCjr internal memory is

allocated.   The /s:  switch controls

the  amount  of   memory   used   for

RAMDrive,   and   the   /v:    switch

controls the amount  of  memory  used

for  the  video  memory  buffer.

If the /s: switch  is  not  specified

then MEMORY.SYS automatically creates

a RAMDrive large enough to  fill  the

PCjr internal  memory.   This  allows

applications to execute out  of  fast

PCjr Booster memory.

If the /v: switch  is  not  specified

then MEMORY.SYS will  allocate  16 KB

for video memory.  This allows you to

run all applications designed for the

IBM PC and XT.   To run PCjr enhanced

video mode applications specify /v:32

To make the PCjr always start up with

an 80 column display, simply  specify

the /w option to MEMORY.SYS

---

Running Flight Simulator

Microsoft Flight  Simulator  has  its

own  routines  for  allocating memory

and will not run  if  the  CONFIG.SYS

file  contains the memory.sys command

line.   Therefore,  make   sure   the

CONFIG.SYS  file  contains only these

command lines:

device clock.sys

device mouse.sys

These  command lines are contained in

the CONFIG.SYS and  CONFIG3.SYS  file

on the PCjr Booster disk.