NINTENDO HARDWARE REGISTERS

$2000 (PPU_Ctrl0)	PPU Control Register 0

bit 7: VBlank NMI enable

if 1, enables VBlank NMIs.  Read $2002 to clear NMI after each VBlank.

bit 6: Slave mode (MUST BE 0) 0=master, 1=slave.

bit 5: Double sprite height

set to 1 to use 16-pixel tall sprites.  The least significant bit of

the character number of the sprite is ignored, and the character

thus specified and the next are stacked, top/bottom.

bit 4: Background character set address

if 0, background characters are stored in $0000-$0FFF, or if 1, they

are stored in $1000-$1FFF.

bit 3: Sprite character set address

if 0, sprite characters are stored in $0000-$0FFF, or if 1, they are

stored in $1000-$1FFF.

bit 2: increment mode for PPU memory access

if 0, the PPU Address Register ($2006) will be incremented by 1 for

each PPU read or write.  If 1, the address will increment by 32.

bit 1: MSB of vertical scroll offset

if 1, vertical scrolling will extend 240 pixels further down.  This

can access a second vertical page.  Note: do not set the vertical

scroll register to a value 240-255, use this bit instead.

bit 0: MSB of horizontal scroll offset

if 1, horizontal scrolling will scroll another 256 pixels to the

right.  This can access a second horizontal page.

$2001 (PPU_Ctrl1)	PPU Control Register 1

bit 7: Sets background to blue.

bit 6: Sets background to green.

bit 5: Sets background to red.

bit 4: Sprite enable

if set to 0, sprites will be blanked.  Always blank sprites before

writing or reading PPU memory, or changing sprite parameters.

bit 3: Background enable

if set to 0, the background will be blanked.  Always blank background

before writing or reading PPU memory.

bit 2: Sprite leftmost column enable

if set to 0, blanks leftmost column of sprites.

bit 1: Background leftmost column enable

if set to 0, will blank leftmost column of background characters.

this can elimate problems with scrolling.

bit 0: Monochrome mode

write a 1 here to switch to a monochrome display.  Tint settings will

still tint the display.

$2002 (PPU_Status)	PPU Status Register

bit 7: VBlank occured

Set to 1 when a VBlank occurs.  Any VBlank NMI routine must clear

this bit by reading sr before another VBlank can occur.

bit 6: collision detection

This bit is set to 1 when any pixel in sprite 0 overlaps a pixel on

the background screen.  It remains set until the end of a VBlank.

Reading this register also sets the Scroll Register ($2005) so that the

next write will be a horizontal scroll value, and sets the PPU Address

Register so that the next write will be the high byte.

$2003 (Sprite_Addr)	Sprite Attribute Memory Address Register

Determines where in sprite attribute memory a DMA will start, or where

in sprite memory a read/write from $2004 will be performed.  If you

change one attribure byte of a sprite, change all four.

$2004 (Sprite_Data)	Sprite Attribute Memory

Read or write to this address to access sprite attribute memory, after

first setting the address to read/write to with $2003 (oamar).  After

each access, $2003 increments by one.

$2005 (Scroll)		Scroll Register, Horizontal and Vertical

The first write to scchv specifies horizontal scroll displacement, and

the second, vertical.  To set vertical scroll larger than 240, use bit

1 of ctrl_0 ($2000).  Horizontal scroll can be changed on the fly, but

vertical scroll can only be changed during a VBlank.  To change vertical

scroll during display time, change the base display address in $2006.

$2006 (VRAM_Addr)	Video RAM Address Register, High and Low Bytes

Write here to set the address for accessing PPU memory (storing high

byte first).  The first byte read from $2007 must be discarded.

Blanking must be set before using $2006 or $2007.  During display time,

$2006 holds the current display address.  If written to during display,

vertical scroll will be ignored, so you must set $2006 to take this into

account.  Before the end of each VBlank, write a pair of zeros here to

restart display at the correct offset.

$2007 (VRAM_Data)	Video RAM Data Register

After setting the address with $2006, and setting the autoincrement mode

with $2000, data may be written or read from the PPU here.  $2006 is

automatically incremented by the proper amount (1 or 32) every read or

write.  When reading, the first byte is invalid, and may be discarded.

$4000 (WRA0)		Duty, Nonstop/Timed Flag, Amplitude/Envelope for Sound A

$4004 (WRB0)		Duty, Nonstop/Timed Flag, Amplitude/Envelope for Sound B

bits 7-6: Specifies duty cycle (waveform).  00=12.5% positive,

01=25% positive, 10=50% positive (pure square), 11=75% positive.

bit 5: 0=timed mode (turns off sound when length counter reaches 0).

1=nonstop mode (sound is continuous, envelopes repeat).

bit 4: 0=envelope mode, 1=direct amplitude mode (see bits 0-3)

bits 3-0: Envelope step speed - 1 (in 1/4 frames) or amplitude setting.

$4001 (WRA1)		"Sweep" (ramping pitch) Sound Register for Sound A

$4005 (WRB1)		"Sweep" (ramping pitch) Sound Register for Sound B

bit 7: Sweep enable.  Set bit 3 to 1 (subtract) when sweep off (0).

bits 6-4: Sweep speed - 1 in quarter frames (60ths of seconds: .25 to 2)

bit 3: add/subtract mode (shift frequency left/right).

0=add (pitch decreases), 1=subtract (pitch increases).  In subtract

mode, sound won't stop unless frequency can be shifted below 7 (which

may not be possible if number in bits 0-2 is too high).

bits 2-0: Number of bits to shift frequency each sweep cycle (0=none).

$4002 (WRA2)		Low 8 Bits of Sound A (square wave) Frequency Code

$4006 (WRB2)		Low 8 Bits of Sound B (square wave) Frequency Code

Can be modified while sound running.  If below 7, sound turns off.

frequency in hertz is 1789772.5/(16 x (frequency code+1)).

$4003 (WRA3)		High 3 Bits Of Sound A Frequency Code, Length Code

$4007 (WRB3)		High 3 Bits of Sound B Frequency Code, Length Code

bits 7-3: Code for length of sound.  When this byte is written to,

counter starts with given value (so high bits of frequency can NOT

be modified while timed sound is running, or sound will restart).

bits 2-0: High bits of sound A frequency.  If high bit (2) is set, and

sweep mode is "add", sound turns off (whether sweep enabled or not).

$4008 (WRC1)		Nonstop/Timed Flag, Envelope Length Data for Sound C

bit 7: If 0, shortest of two timers is used.  if 1, sound is continuous

if length counters are not both 0 (which always turns sound off).

bits 6-0: length - 1 in quarter frames (.25 to 32)

$400A (WRC2)		Low 8 Bits of Sound C (triangle wave) Frequency Code

Frequency in hertz is 1789772.5/(16 x (frequency code+1)).  Does NOT

turn off sound if frequency code is below 7.

$400B (WRC3)		High 3 bits of Sound C Frequency Code, Length Code

bits 7-3: Code for length of sound (as in WRA3 and WRB3, above).  Sound

is started by writing to this byte.

bits 2-0: High bits of sound C frequency.

$400C (WRD0)		Nonstop/Timed Flag, Amplitude/Envelope for Sound D

bit 5: 0=timed mode (turns off sound when length counter reaches 0).

1=nonstop mode (sound is continuous, envelopes repeat).

bit 4: 0=envelope mode, 1=direct amplitude mode (see bits 0-3)

bits 3-0: Envelope step speed or direct amplitude setting.

$400E (WRD2)		Maximum Frequency Code for Sound D (noise)

bits 3-0: Determines the "sample clock frequency" (maximum frequency of

random frequencies making up noise).

$400F (WRD3)		Length Code for Sound D

bits 7-3: Code specifying length of sound D (as for sounds A, B, and C).

Writing to this bytes starts sound playing.

$4010 (WRE0)		Sample Speed, Repeat/Single Shot, IRQ Enable for Sound E

bit 7: set to 1 to enable an IRQ at the end of each sample playback.

bit 6: 1=repeat sound upon completion, 0=play sound once.

bits 3-0: Specifies sample playback speed (decoded by lookup table).

$4011 (WRE1)		Sound E (Delta Mod) Direct Data Output

Bits 6-1: Holds data read from memory to be sent to the A/D converter.

Samples are stored as single bits, each of which increments (1) or

decrements (0) the amplitude of the waveform.

$4012 (WRE2)		Initial Address of Sound E Waveform Data

Actual address is computed by $C000+(WRE2*16).

$4013 (WRE3)		Length of Sound E Waveform Data

Store length of sample here.  Length in bytes is (16*WRE3)+1.  The

duration of the sound will depend on the sample speed (in WRE0).

$4014 (Sprite_DMA)	Start Sprite DMA At Specified Page

Writing a number to this address specifies the page number (high byte of

address) from which sprite attribute DMA will start, and starts the DMA.

$4015 (Sound_Enable)	Sound Enable/Length Flag, IRQ Flag Register

bit 7: reads setting of IRQ flag for sound D.

bit 4: enables sound E (delta modulated samples).

bit 3: enables/reads sound D (noise).

bit 2: enables/reads sound C (triangle wave).

bit 1: enables/reads sound B (square wave), as above for sound A.

bit 0: set to 1 to enable sound A (square wave), or 0 to disable.  When

read, this bit shows if the length counter for sound A is running.

$4016 (Joystick1)	Game Controller Port 1

$4017 (Joystick2)	Game Controller Port 2

Write a 1 then a 0 to the port, then each subsequent read will return

a bit specifying the state of a certain button on the controller (1 is

pressed, 0 is not pressed).  Write $40 to $4017 during initialization

to disable CPU LSI internal IRQ.

bit 7: right		bit 3: start button

bit 6: left		bit 2: select button

bit 5: down		bit 1: button B

bit 4: up		bit 0: button A

NINTENDO CPU MEMORY MAP

$0000-$07FF:  2K bytes RAM, in Nintendo base unit.

$2000-$2007:  PPU Registers.

$4000-$4017:  Sound, game controller, and IRQ control registers.

$6000-$7FFF:  Up to 8K optional RAM expansion on cartridge.

$8000-$BFFF:  16K of bank-switched ROM on cartridge.

$C000-$FFFF:  16K of fixed ROM on cartridge (sampled sound can only be here).

NINTENDO PPU MEMORY MAP

Bank Switchable ROM:

$0000-$0FFF:  Character set for sprites or background.

$1000-$1FFF:  Alternate character set for sprites or background.

RAM:

$2000-$23BF:  Character matrix for display page 1.

$23C0-$23FF:  Area colors for display page 1.

$2400-$27BF:  Character matrix for extra horizontal screen (if enabled).

$27C0-$27FF:  Area colors for extra horizontal screen.

$2800-$2BBF:  Character matrix for extra vertical screen (if enabled).

$2BC0-$2BFF:  Area colors for extra vertical screen.

$2C00-$2FBF:  Character matrix for page 4 (if enabled).

$2FC0-$2FFF:  Area colors for page 4.

$3F00-$3F1F:  Color palette for sprites and background.

SPRITE ATTRIBUTE MEMORY

64 Sprites are stored in this 256-byte area, taking up 4 bytes each.  A page

of RAM is used to hold the data, and the programmer must issue a DMA with

the page number specified to transfer the data to the PPU.

byte 0:  Vertical position on screen (ignores scrolling).

byte 1:  Character number in sprite character set.  If in double-height sprite

mode, low bit is ignored, and next character is stacked below.

byte 2:  Attribute:

bit 7: set to 1 to flip sprite horizontally.

bit 6: set to 1 to flip sprite vertically.

bit 5: sets priority (if 1, background covers sprite).

bit 0-1: chooses sprite palette to use.

byte 3:  Horizontal position on screen (ignores scrolling).

CHARACTER MATRIX DEFINITION

The background screen is broken down into 32x30 characters, each 8x8 pixels.

The screen background is defined by a matrix of 960 bytes, each holding the

number of the character.  The bytes are stored linerally, the first byte

holding the character in the upper-left corner of the screen.

CHARACTER SET MEMORY

Each character takes 16 bytes to define.  The first 8 bytes consists of the

low bits of the color of each pixel for each line of the 8 lines of the

character, and the last 8 define the high bits.  The actual color depends on

the palette specified in the area color section of PPU RAM.  If both bits are

0, however, the color is always transparent (black).

AREA COLOR DEFINITION

Each 2x2 block of characters chooses colors from one of four palettes.  A byte

is used to hold the palette numbers for each 2x2 block of color blocks (one

byte for each 4x4 character block).  Bits 0-1 are used for the upper left 2x2

character block in the 4x4 block, bits 2-3 for the upper right, bits 4-5 for

the lower left, and bits 6-7 for the lower right block.  64 bytes, arranged in

an 8x8 matrix, define the palettes for the entire screen.

PALETTE DEFINITION

A palette holds 4 colors that correspond to the bits in the character

definition.  The color is made up of 6 bits, the low 4 being the base color,

and the high 2 being the intensity.  Intensity 0 is the darkest, and 3 is the

lightest.  The usable colors are listed below:

0: Gray	(white)			8: Yellow

1: Blue				9: Green (yellow)

2: Blue (purple)		A: Green

3: Purple			B: Green (blue)

4: Pink				C: Cyan

5: Red				D: Grey 2 (use only with intensity 3)

6: Orange			E: Black 1 (use only with intensity 3)

7: Brown			F: Black 2 (use only with intensity 0)