INTRODUCTION

NOBOUNDS is a program designed to be used for the NES development of

sprites and frames, making it possible to do overlaying of sprites within

a frame and without square boundaries. It receives its data from EGA IFF

files, usually generated by programs such as DPAINT.

The EGA or 16 color VGA file must be set up in the following format:

COLORS 0-2:  FIRST NES PALETTE

COLORS 3-5:  SECOND NES PALETTE

COLORS 6-8:  THIRD NES PALETTE

COLORS 9-11: FOURTH NES PALETTE

COLOR  14:   TRANSPARENT COLOR USED FOR BACKGROUND OF FRAMES

COLOR  15:   SCREEN DOES NOT CARE

ALL OTHER COLORS REGARDED AS NON-USABLE AND NOBOUNDS WOULD IGNORE THEM

OR ERROR OUT.

REFERENCE TO COLORS IS NOT A REFERENCE TO TRUE COLORS BUT A REFERENCE

TO THE EGA/VGA PALETTE ORDER.

ALL objects must be drawn on 8x8 pixel boundaries and no object or

group of 8x8 can be directly touching another group. The seperation must

be at least 8 pixels. If using NOBOUNDS 16 the 16 pixel bounds apply.

EXAMPLE:

Object 1          Object 2

_______          _______

|_|_|_|_|    ^   |_|_|_|_|

|_|_|_|_|    ^   |_|_|_|_|

|_|_|_|_|    ^   |_|_|_|_|

^

Must be 8 pixels

Object 1

_____

|_|_|_|

|_|_|_|_____

|_|_|_|

Will work  |_|_|_|

Object 2

INSTRUCTIONS

To run the NOBOUND program, simply type the command NOBOUNDS at the

PC command line. A filename must also be passed which represents either

the name of an IFF EGA file to be ripped apart or a work file to continue

working on.

EXAMPLE:

NOBOUNDS FILENAME

This would envoke the NOBOUNDS program, in which case it would try to

load a file FILENAME.LBM. If the file exists, it would try to use this file

as an object file for creating NES sprites. After successfully ripping the

screen, it would then go to the editor where the frames could be built

up from the pieces that the ripper generated.

EXAMPLE:

NOBOUNDS FILENAME -

With this command line, NOBOUNDS would immediately try to load the work

file FILENAME.NBS. If the file is found, it will take you directly to the

NOBOUNDS editor where you can continue to edit the animation frames which

had been previously generated.

COMMANDS

The following is a list of commands available in the NOBOUNDS editor:

S:         WILL SAVE AN SPD FILE OUT (ARTEMIS SPD FORMAT).

L:         WILL LOAD THE WORK FILE FILENAME.NBS (BE CAREFUL! IF FILE NAME

DOES NOT EXIST, PROGRAM WILL HALT).

>.:        ADVANCES FRAMES BY ONE

<,:        DECREMENTS FRAMES BY ONE

SPACE:     INSERTS NEW PIECE INTO GAME, GIVING TOP PRIORITY-CAN ONLY BE USED

WHEN FLAG IS EQUAL TO 0.

ARROWS:    MOVES THE CURRENT PIECE BEING POINTED TO AROUND WITHIN A FRAME.

+,-KEYS:   USE TO SELECT THE PIECE

F1:  DECREMENTS THE BUFFER COUNTER, ALLOWING YOU TO REPOSITION A PIECE

WHICH IS ALREADY IN THE FRAME.

F2:  INCREMENTS THE BUFFER POINTER BUT NOT BEYOND MAXIMUM CURRENT BUFFERS

WITHIN THAT FRAME.

INS: USED TO INSERT A PIECE WITHIN THE FRAME BETWEEN ALREADY POSITIONED

PIECES.

DEL: USED TO DELETE THE PIECE AT THE CURRENT BUFFER POSITIONS.

ESC: USED TO ESCAPE THE PROGRAM WITHOUT SAVING.

YOU MAY ALSO SELECT A PIECE BY SIMPLY TYPING IN A 2 DIGIT NUMBER.

U:   UNDO FUNCTION-THIS IS USED TO UNDO THE LAST CHANGE.

F9:  THIS IS USED TO MARK A FRAME TO BE COPIED TO ANOTHER LOCATION.

X:   THIS IS USED TO COPY THE FRAME PREVIOUSLY MARKED TO THE FRAME

CURRENTLY BEING POINTED TO.

F10: THIS IS USED TO CLEAR A FRAME.

W:   THIS IS USED TO WRITE OUT AN .ASM FILE TO DISK.

.SPD FILE FORMAT

This is generated by using the NOBOUNDS save command. The format is as

follows:

0-5F =      COLOR SETS

60-405F =   CHARACTER SET

4060-415F = CHARACTER COLOR SET

4160-416F = EGA TO NES COLOR CONVERSION TABLE

4170-A1AF = 64 FRAME DEFINITIONS, EACH USING 385 BYTES, A MAXIMUM OF 64

CHARACTER DEFINITIONS EACH.

The first byte is a count of characters used. Each character takes 6

bytes to define-2 byte X position, 2 BYTE Y position, charater number and

color/flip status.

.NBS FILE FORMAT

The .NBS file is generated when using the Q function to quit the

program. This is a NOBOUNDS workfile, format is as follows:

16383 BYTES OF PIECE DATA.

512 BYTES OF PIECE INDEX.

2 BYTES NUMBER OF PIECES.

16383 BYTES OF FRAME DATA.

1200 BYTES OF CURRENT SCREEN MAP.

4096 BYTES OF NES CHARACTER SET.

96 BYTES OF NES COLOR SET.

16384 BYTES OF EGA/VGA CHARACTER SET.

256 BYTES OF NES CHARACTER COLORS.

16 BYTES OF EGA TO NES CONVERSION COLORS.

2 BYTES NUMBER OF CHARACTERS.

960 BYTES ANIMATION LIST DATA.

48 BYTES VGA/EGA COLOR PALETTE REGISTER DATA.

.ASM FILE FORMAT

The .ASM file is saved out when using the write command. This is a

file that could be included into a program.

Included is a word table pointing to the frames.

Frame data is set up in the following manner:

FIRST BYTE = NUMBER OF CHARACTERS

EACH CHARACTER TAKES 4 BYTES TO DEFINE

Y FRAME OFFSET FROM UPPER LEFT CORNER OF FRAME

CHARACTER NUMBER

COLOR/FLIP (COLOR = LOW NIBBLE, FLIP IS IN HIGH NIBBLE)

X FRAME OFFSET FROM UPPER LEFT CORNER OF FRAME

NOTE

If filenames are named in the following format, the program is

designed to handle multiple files.

EXAMPLE:

FILENAME1.LBM

FILENAME2.LBM

FILENAME3.LBM

************************* IMPORTANT ************************************

Revisions for VERsion 4.0.

All loads and saves ask for filenames. Valid names are 8 chars or less

of alpha characters only. Backspace will delete, and hitting return

without a filename will abort the load or save. The format of the .NBS

file has changed. However there is a function that will enable the user

to load an old .NBS file into the new program and then the user can

save out the file. The file will then be in the new format.

L = load a .NBS entire file.

I = Send a .NBS file to disk.

P = Load only pieces from a .NBS file.

K = Load only frames from a .NBS file.

R = Load only character set from a .NBS file.

***********************************************************************

Revisions for version 5.0

There have been several added features for 5.0. One big change is the

animation scheme addition. Here is the new stuff.

F3:  FROM THE FRAME MAKER WILL TAKE THE USER TO A CHARACTER EDIT UTILITY.

F4:  FROM THE FRAME MAKER WILL TAKE THE USER TO A PIECE MAKER UTILITY.

A:   FROM THE FRAME MAKER WILL TAKE THE USER TO THE ANIMATION EDITOR.

CHARACTER EDITOR

The character editor is crude but usable for making small changes to the

NOBOUNDS file without having to set up or redo the original files. However

if the need to reconvert arises there is no means for saving the changes to

the newly converted data (beware!).

The following is a list of keystroke commands that can be used in the

character editor.

ARROW KEYS:  MOVE EDIT CURSOR WITHIN CHARACTER.

C:           COPY PREMARKED CHARACTER TO THIS CHARACTER.

M:           MARK THIS CHARACTER FOR COPY.

+ -:         SELECT CURRENT CHARACTER.

1,2,3:       SELECT PIXEL COLOR.

4,5,6,7:     SELECT CURRENT PALLATTE.

F1:          CLEAR THE CHARACTER.

SPACE:       PLACE PIXEL IN CHARACTER.

BACKSPACE:   DELETE PIXEL FROM CHARACTER.

ESC:         RETURN TO FRAME MAKER.

PIECE EDITOR

The piece editor is also somewhat crude. However it is usable for adding

pieces and flipping data etc.

The following is a list of keystroke commands to use within the PIECE MAKER.

ARROW KEYS:  MOVE EDIT CURSOR WITHIN PIECE.

+ -:         SELECT CURRENT CHARACTER (BRUSH).

1,2,3:       SELECT PIXEL COLOR (NOT REALLY USED).

4,5,6,7:     SELECT CURRENT PALLATTE.

F1:          ADD NEW PIECE TO END OF DATA.

SPACE:       PLACE CHARACTER IIN PIECE.

BACKSPACE:   DELETE CHARACTER FROM PIECE.

X:           FLIP CHARACTER X.

Y:           FLIP CHARACTER Y.

F9:          DELETE ENTIRE PIECE.

F10:         INSERT A NEW PIECE IN BETWEEN DATA.

ESC:         RETURN TO FRAME MAKER.

ANIMATION EDITOR

The animation editor will allow the user to define 2 animation structures

holding  8 animation sequences of 20 steps each. The user can then download

the animations to the NES or save them out to the .ASM or .NBS files. To

see the animations on the NES you will first need to load the NES with the

SPEDNDS driver utility.("SPEDNDS>HEX")

Arrow keys: Move cursor position within animation editor.

+ - keys:   Adjust the values under the cursor position.

D:          Down load animation sequences to the NES.

ESC:        Return to frame maker.

0-7:        Select an animation sequence to edit.

P:          Select an animation structure to edit.

L:          Use this to set a label for an animation list.

W:          Use this to write out the current animation structure.

To an .ANM file. This is an animation list you can include

into your program.

The list format is as follows:

frame#,delay,x offset,y offset